Project No. 03-14

AIRPORT PASSENGER CONVEYANCE

SYSTEM USAGE/THROUGHPUT

ACRP 03-14

FINAL REPORT

Prepared for

ACRP

Transportation Research Board

of

The National Academies

TransSolutions

Clemson University

Kimley-Horn and Associates

Fort Worth, Texas

December, 2011

ACKNOWLEDGMENT OF SPONSORSHIP

This work was sponsored by one or more of the following as noted:

 American Association of State Highway and Transportation Officials, in cooperation with the Federal Highway Administration, and was conducted in the National Cooperative Highway Research Program,

 Federal Transit Administration and was conducted in the Transit Cooperative Research Program,

Federal Aviation Administration and was conducted in the Airport Cooperative Research Program,

 Research and Innovative Technology Administration and was conducted in the National Cooperative Freight Research Program,

 Pipeline and Hazardous Materials Safety Administration and was conducted in the Hazardous Materials Cooperative Research Program,

which is administered by the Transportation Research Board of the National Academies.

DISCLAIMER

This is an uncorrected draft as submitted by the research agency. The opinions and conclusions expressed or implied in the report are those of the research agency. They are not necessarily those of the Transportation Research Board, the National Academies, or the program sponsors.

TABLE OF CONTENTS

Executive Summary	1

1.	background	2

1.1.	Project Overview & Objectives	2

1.2.	Organization of This Report	2

1.3.	Guidebook and Database	3

2.	LITERATURE REVIEW	4

2.1.	Planning for Passenger Conveyance Systems at Airports	4

2.2.	Pedestrian Behavior and Walking Distance	5

2.3.	Level of Service of Pedestrian Facilities	7

2.4.	Passenger Conveyance Systems	7

2.5.	Capacity of Conveyance Systems	8

2.6.	Automated People Mover Systems	9

3.	Catalog of passenger conveyance systems	10

3.1.	Planning Assumptions and Guidelines	10

3.2.	System Physical Characteristics	11

4.	Data Collection	20

4.1.	Purpose	20

4.2.	Data Obtained and Data Collection Method	20

4.3.	Selection of the Five Airports for Data Collection	20

4.4.	Periods of Data Collection	24

4.5.	Airport Approval/Coordination	25

4.6.	Initial Data Collection Plan	25

4.7.	On-Site Observations at Each Airport	26

4.8.	Passenger Behavior	51

4.9.	Maintenance Costs	52

5.	Data Analysis	54

5.1.	Escalators and Elevators	54

5.2.	Moving Walkways	58

5.3.	Summary of Other Database Components	60

Glossary of passenger conveyance system terms	62

REFERENCES	64

LIST OF FIGURES

Figure 1. Typical Area Required for Standee	12

Figure 2. ThyssenKrupp TurboTrack	17

Figure 3. High-Speed Moving Walkway Installed at Toronto Pearson International Airport	17

Figure 4. Sample of PDA Data Collection Screen	20

Figure 5. Decision Matrix for Airport Data Collection Sites	23

Figure 6. ATL Terminal Layout	26

Figure 7. Escalators at APM Station	27

Figure 8. Three Escalators from APM at Main Terminal	27

Figure 9. Passenger Queue for Escalator to APM	28

Figure 10. Passenger Queue for Escalator from APM Station	28

Figure 11. Passenger Queue for Escalator from APM	29

Figure 12. Passenger Queue for Elevator in Concourse	29

Figure 13. Wheelchair Storage Near Check-In (Delta)	30

Figure 14. Wheelchair Storage Near Check-In (AirTran)	30

Figure 15. Storage for Traditional Wheelchairs	30

Figure 16. CLT Terminal Layout	31

Figure 17. Passenger Choice of Moving Walkway	32

Figure 18. Moving Walkways in the Middle of a Corridor	33

Figure 19. Moving Walkways on Side of a Corridor	33

Figure 20. Moving Walkways, Escalator, and Electric Passenger Cart	33

Figure 21. Signage for Two Lengths of Moving Walkways	34

Figure 22. Wheelchair Storage Near Gate	34

Figure 23. DFW Terminal Layout	35

Figure 24. Moving Walkways between Terminals	36

Figure 25. Moving Walkways in Center Section of Terminal D	37

Figure 26. Moving Walkways on Side of Corridor in Terminal D	37

Figure 27. Escalators and Elevator in Skylink Station	38

Figure 28. Elevator at Far End of Skylink Station	38

Figure 29. One of Three Escalators Out of Service	39

Figure 30. Electric Carts at DFW	40

Figure 31. DEN Terminal Layout	41

Figure 32. Collecting Data at the Security Screening Checkpoint Exit	42

Figure 33. Passengers on Escalators and Stairs to APM	42

Figure 34. Pair of Moving Walkways in DEN Concourse	43

Figure 35. Concourse B with Two Moving Walkways in Each Direction	43

Figure 36. Moving Walkways with Two People Abreast	44

Figure 37. Moving Walkway with Three People Abreast	44

Figure 38. Passengers Choose Moving Walkways or Corridors	45

Figure 39. Two Escalators in Each Direction at APM Station	45

Figure 40. Queue Forms at APM Station Escalator	46

Figure 41. Cart Storage and Recharging Area	46

Figure 42. Temporary Cart Storage in Boarding Lounge	47

Figure 43. MCO Terminal Layout	48

Figure 44. Queue at Elevators at Baggage Claim	48

Figure 45. Queue Developing at Top of Escalators	49

Figure 46. Children with Escalator Boarding Difficulty	49

Figure 47. Escalator and Stairs to Baggage Claim	50

Figure 48. Signage Directing Passengers to Specific Escalator	50

Figure 49. Escalators with Different Destinations	51

Figure 50. SEA Escalator Annual Maintenance Costs	53

LIST OF TABLES

Table 1. Passenger and Bag Space Requirement	11

Table 2. Elevator Characteristics	13

Table 3. Escalator Characteristics	15

Table 4. Moving Walkway Characteristics	16

Table 5. Wheelchair Lift Characteristics	19

Table 6. ATL Characteristics	26

Table 7. CLT Characteristics	31

Table 8. DFW Characteristics	35

Table 9. DEN Characteristics	41

Table 10. MCO Characteristics	47

Table 11. Passenger Conveyance Annual Maintenance Cost, Per Unit	53

Table 12. Average Escalator Board Rates (in persons per minute) by Airport	54

Table 13. Average Escalator Board Rates (in persons per minute) by Airport and Direction	54

Table 14. Average Escalator Board Rates (in persons per minute) by Airport and Tread-Width (in inches)	55

Table 15. Average Escalator Board Rates (in persons per minute per escalator)  by Airport and Number of Escalators	55

Table 16. Average Escalator Board Rates (in persons per minute per escalator) by Airport,  Number of Escalators, Direction, and Width (in inches)	56

Table 17. Elevator Board Times (in seconds) and Passenger Characteristics by Airport	56

Table 18. Passenger Vertical Conveyance Mode Choice (in percentage of persons) by Airport	57

Table 19. Passenger Vertical Conveyance Mode Choice (in percentage of persons) by Airport and Direction	57

Table 20. Number of Rollers by Airport and by Passenger Vertical Conveyance Mode	58

Table 21. Moving Walkway Board Rates (in persons per minute) by Airport and Width (in inches)	59

Table 22. Passenger Horizontal Conveyance Mode Choice (in percentage of persons) by Airport	59

Table 23. Moving walk Mode Choice and Number of Rollers by Airport	59

ACKNOWLEDGMENTS

This research was conducted under the Airport Cooperative Research Program (ACRP) Project 03-14 by TransSolutions, Kimley-Horn Associates, and Clemson University.  Principal Investigator was Belinda Hargrove (TransSolutions), assisted by many other TransSolutions staff members. Key contributors from Kimley-Horn Associates were Jennifer Bihl, Jill Capelli, and Adam Novak, and key researchers from Clemson University were Dr. Kevin Taaffe and Ping-Nan Chiang.

The authors are very grateful to the guidance and patience of the TRB staff, especially Lawrence (Larry) Goldstein, Senior Program Officer. The project panel, led by Mr. Theodore (Ted) Kitchens, also offered valuable guidance in the successful completion of the project.

As part of this project, the research team collected passenger conveyance choice, usage, and throughput data at five U.S. airports. The usefulness of this research project’s output is entirely dependent on the cooperation and support of personnel at those five airports. Specifically, we want to thank:

- Charlotte Douglas International Airport – Roy Johnson and Jennifer Long

- Dallas/Fort Worth International Airport – Andy Bell and Justin Thomason

- Denver International Airport – Reggie Norman

- Hartsfield-Jackson Atlanta International Airport – Jim Drinkard, Ted Kitchens, and the numerous escorts from the Planning Department

- Orlando International Airport – Rob Brancheau and Art Devine

Special thanks to numerous staff of the organizations who assisted in the completion of the project through preparing the data collection instruments, collecting the data, and consolidating the data into a useful tool for the industry.

# Executive Summary

This Final Report of ACRP 03-14 project summarizes the research conducted to assist airports for planning and evaluating passenger conveyance systems at airports. The primary conveyances included in this work include elevators, escalators, and moving walkways.

Accompanying this document are two key deliverables:

- “Airport Passenger Conveyance Systems Guidebook” which provides best practices, design considerations, and decision-making frameworks for implementing passenger conveyance systems. The Guidebook includes detailed information on elevators, escalators, moving walkways, as well as considerations for passenger assist vehicles (e.g., electric courtesy carts) and wheelchairs.

- Database of passenger conveyance systems usage data, which includes the actual throughput and usage/choice data obtained on-site by the research team at five North American airports; the user can query the database by airport transition area. Also in the database is a planning tool where the user can input airport-specific information, compare the inputs to the collected data from the five airports, and compare options for implementing conveyance systems to meet the projected demand at their airport. Available conveyance equipment is also documented in the database. A database user guide is documented in the Guidebook.

The research findings provide valuable information regarding practical throughput or capacity of conveyance systems in airport environments.  Stated capacity values are seldom achieved in airports due to the surges of passenger demand as well as additional space requirements for luggage. This project also documented the choices made by passengers when provided options for vertical transition (elevator, escalator, stairs) and horizontal transition (corridor, moving walkway). This information assists planners with assessing the expected use of passenger conveyance systems in airport designs.

# background

## Project Overview & Objectives

The primary objective of this research effort was to prepare a comprehensive guidebook to serve as a decision-support tool for planning, designing, and evaluating passenger conveyance systems at airports.  The guidebook includes the following components:

- Design considerations for including passenger conveyance systems in an airport environment

- A decision-making framework with user instructions, that includes:

  - Checklists for planning, design, selection, and feasibility of specific passenger conveyance components at an airport

  - Key steps for implementing a passenger conveyance system

  - Requirements for implementing, operating, and maintaining the passenger conveyance system.

Recognizing that another ACRP project has developed planning guidelines for Airport People Mover Systems (APMs), published as ACRP Report 37 (2010), this project will not consider APMs.

A secondary objective for this research was to compile the analyzed data from passenger conveyances collected at five North American airports into a database.

The benefit of these items to the aviation community is the availability of a set of current conveyance options, usage estimates, and planning criteria specific to North American airports, for planners, architects and engineers.  This guidebook and its decision-making framework provides assistance to airport planners and operators when considering conveyances to facilitate the movement of passengers and reduce walking/transit times through airports, resulting in North American airports with efficient passenger transit with a positive passenger experience.

## Organization of This Report

This Report summarizes the research conducted during the project for understanding how to plan for passenger conveyance systems at airports.  Specifically, the Report includes the following:

- Chapter 1, Introduction – provides the reader with an overview of the research project and the major deliverables of the project.

- Chapter 2, Literature Review – provides background information the research team gathered when the project was initiated on the available planning tools and research in the area of pedestrian conveyance systems.

- Chapter 3, Passenger Conveyance Systems/Characteristics – provides an overview of the available passenger conveyance systems, manufacturers, system characteristics, and capacity estimates.-

- Chapter 4, Data Collection at Five Airports – describes the selection of five airports for the data collection and a description of the data obtained at each airport, including observations that are useful for other airports

- Chapter 5, Data Analysis Summary – summarizes some key data obtained during this project at the five airports

## Guidebook and Database

Major deliverables from this project include the “Guidebook for Planning Passenger Conveyance Systems at Airports” and the accompanying database. The Guidebook assists airport planners and operators in planning, designing, and operating passenger conveyance systems at an airport.  The Guidebook contains practical information and tips for operations/maintenance. The Guidebook includes best practices for planning/designing airport passenger conveyance systems, as well as descriptions of physical characteristics of the systems, passenger difficulties in using different types of systems, capacity/throughput considerations, and general maintenance information. Design considerations include information on airport layout and various passenger characteristics,

Also, the Guidebook incorporates information from the data collection to help airports evaluate when passenger conveyance systems should be considered for the specific airport environment.  It has sufficient information to enable an airport to, at least on a high-level, estimate the passenger service improvement that can be realized from implementing a new passenger conveyance. Finally, the Guidebook includes a decision-making framework to assist planners with evaluating types of conveyances useful in specific situations.

The database includes all the data collected on-site at the five airports during this project, an inventory of available conveyance systems, as well as a planning tool. Users can input airport-specific information along with passenger demand to obtain conveyance requirement estimates. Users can also query the data obtained at the five airports to compare their input and output data with the data from other airports.  The interactive planning tool allows the user to set unique demand levels and compare these against what certain facilities were able to obtain in throughput (for example) using their airport-specific configurations.

# LITERATURE REVIEW

In recent years, airports have been challenged to continue to provide good passenger experience despite increasingly constrained budgets.  Decisions on Capital Improvement Programs and asset management need good data to guide expenditures of limited and quite possibly, shrinking, capital dollars. Meanwhile, to meet often increasing passenger demand, airports are becoming larger and airport terminal facilities continue to increase in size. One well-known example is the introduction of the Airbus A380 and the new requirements it poses in terminal planning (Pfurr 2006). Airports unable to increase capacity are faced with overcrowded terminals, leading to safety/risk issues especially at peak times.

In general, planners, designers, and operators of airports face substantial challenges in how to move their passengers faster and more efficiently. To achieve acceptable passenger walking distances, aircraft-to-aircraft transfer times, and overall transit times in terminals, several passenger mobility technologies are commonly used. These technologies include moving walks, passenger assist vehicles, buses and automated people movers (APMs). We introduce related literature as it pertains to five main categories: passenger conveyance planning, pedestrian behavior and walking distance, level of service of pedestrian facilities, passenger conveyance systems, and capacity of conveyance systems. While APM systems are covered within ACRP 03-06 (published as ACRP Report 37) and ACRP 03-07, we briefly mention a few key research examples.

## Planning for Passenger Conveyance Systems at Airports

There are many sources describing the process and guidelines for airport terminal planning (see, e.g., Ashford et al. 1996, Dempsey 2000, Omer 1988). In the Apron and Terminal Building Planning Manual, the Ralph M. Parsons Company (1975) also provides guidance for planning airport apron-terminal complexes. They briefly discuss circulation, however there is little mention concerning the effects of walking distances on passengers and their walking distance preferences. The Federal Aviation Administration (1994) mentions the possibility of installing moving walks, escalators, etc., to make excessive walking distances more tolerable.

In Planning and Design of Airports, Horonjeff et al. (2010) state that walking distance should be examined and considered in the terminal design development. As with other planning and design references, very few insights into acceptable walking distances are provided. Wells (2011) and Odoni and Richard (1992) also mention that airports should consider minimizing walking distances for passengers when designing terminal building space requirements. Another widely used planning guideline is provided by IATA (International Air Transport Association 2004), which suggests a maximum passenger walking distance of 820 – 985 feet (250-300m) unaided and up to 2,133 ft (650m) with moving walks. Delve (2004) mentions that size and positioning of escalators and other people mover systems at airports are very important to minimize the time and distance that passengers travel. He also suggests a strategy for exposing passengers to various revenue-generating sites such as stores and restaurants while proceeding through the terminal.

Design projects are not always focused on improving passenger travel time efficiency. Russell (2004) reviews a project to expand the number of service stands at London’s Gatwick Airport. The focus of this article is on the use of a new passenger bridge that connects the North Terminal with the Pier 6 satellite building. While not specifically designed to reduce passenger travel times, the bridge provides passengers a direct pedestrian link to aircraft, saving an estimated 50,000 coach journeys a year. With 200-ft (61-meter) long moving walks and 33 feet (10) meters between each moving walk, it also provides an enjoyable walking experience for passengers.

When discussing optimal passenger terminal building configurations, de Neufville et al. (2002) mention that moving walks are a relatively inexpensive means to move people through an airport. In a comparison study, Leder (1991) reviews advantages, disadvantages, and limitations of four airport terminal passenger mobility systems: moving walks, electric passenger carts, buses, and automated people movers. In this article, the author suggests that moving walks can be used to aid passenger mobility when the total distance of passenger movement does not exceed 1000 to 1500 ft, and when point-to-point travel along a straight line is acceptable (or allowable). On the contrary, APMs are best suited for route lengths in excess of 1000 ft and with relatively high ridership. APMs can also offer a high level of schedule and trip time dependability due to their exclusive right-of-way.

Buses have proven to be a successful mode of intra-airport transportation due to its flexible route and low operation cost. However, they are often crowded and uncomfortable. Moreover, they usually travel at slower speeds due to the shared right-of-way with vehicular traffic, may have lengthy dwell times in each stop as well as multiple stops. Those reasons often result in low level-of-service ratings by users. A fleet of buses also requires recurring maintenance, increasing an airport’s operating costs.

Electric carts serve an important role in assisting handicapped passengers and offer flexibility that moving walks and APMs do not, mainly because they are maneuverable and do not require an exclusive right-of-way. However, electric carts are often not a viable transportation mode because they operate in mixed traffic with pedestrians. The carts run very quietly such that passengers are not aware of them until the cart is directly behind the passenger. Carts also take up space, effectively reducing the corridor width available to pedestrians, becoming very problematic in narrow corridors.

While APMs and buses do not qualify as passenger conveyance options within the scope of ACRP 03-14, we mention these transport modes simply because they play an integral role in the overall movement of passengers through airports.

## Pedestrian Behavior and Walking Distance

It is well documented that pedestrian behavior (in general as well as specifically within airports) is a very important factor when considering acceptable walking distances. Several articles provide contributions in the interaction between facility design of walking requirements, as well as appropriate walking speeds and distances.

The ability to assess pedestrian behavior based on actual data in real systems cannot be overemphasized. Researchers often analyze the actions of other people in lab conditions for the purpose of action coordination. In order to understand whether such self-relative action perception differs from other-relative action perception, Jacobs and Shiffrar (2005) conduct a design of experiments and suggest that the visual analysis of human motion during traditional laboratory studies can differ substantially from the visual analysis of human movement under more realistic conditions. In contrast, we offer many examples of studies where researchers have studied existing transport systems to more accurately determine (and predict) pedestrian behavior.

Hoogendoorn and Daamen (2005) introduce experimental findings of pedestrian behavior when faced with bottlenecks in flow. Essentially, pedestrians inside such bottlenecks form layers or trails, with a typical separation of approximately 45 cm (17.7 in). This is less than the effective width of a single pedestrian, which is around 55 cm (21.7 in). When quantifying pedestrian movement, Hui et al. (2007) found that walking speed, step size and step frequency all followed normal distributions. Moreover, gender and age significantly affected these three measures, except for walking speed and step size of children and older pedestrians. These results were based on data collected in Beijing, China. Helbing (1991) provided a more specific perspective by presenting a mathematical model for the movement of pedestrians.

Walking distance and walking speed are significant factors when installing automated pedestrian movement systems within airport terminals. Seneviratne (1985) proposes an approach for determining critical pedestrian walking distance. Based on findings from a series of surveys in Calgary, Alberta, they found that the critical pedestrian walking distance distribution is dependent on the classification of the pedestrian. The results show that the best walking distance distribution for most work-based trips follows a gamma distribution and the critical distance is estimated at 796 feet (243 m). This is the same methodology first introduced by Pushkarev and Zupan (1975), where they identified a critical walking distance distribution for urban areas.  They report that average walking distances in central London were more than 2,625 feet (800 meters), whereas those in midtown New York City were 1,719 ft (524 meters). Moreover, Pushkarev and Zupan (1975) state the advantages and limitations when using an escalator and a moving walk. However, they leave the optimal length of a moving walk as an open issue. In order to solve this problem, Bandara and Wirasinghe (1986) and Bandara (1989) develop an analytical model for optimizing pier-type terminal configurations.  They consider an objective function that minimizes the sum of system operational costs and individual user costs to determine the optimal length of the moving walk.

When discussing walking speed, walking distance, and level of service of facilities in public, Fruin (1971) conducted a series of studies on the behavior of pedestrians within transportation terminals. Two studies in particular--conducted at the Port Authority Bus Terminal and at the Pennsylvania Train Station, both located in New York City--observed pedestrian walking speeds under free-flow conditions along with various observable pedestrian characteristics. Among the characteristics included were age, gender, trip purpose, number of bags carried, direction of travel, size of group, and final destination within the terminal. Fruin found that the mean walking speed was approximately 265 ft (80.8 meters ) per minute, with a standard deviation of 50 ft (15.3 meters) per minute. Seneviratne and Wirasinghe (1989) perform a cost analysis with the goal of optimizing airport terminal corridor width.

Finally, Zacharias (2001) discusses acceptable walking distances in city areas and suggests that effective planning decisions be developed through further research. Smith and Butcher (2008) discuss the various conditions that should be taken into account to determine how far people using parking garages should be asked to walk.

There are several works which provide different perspectives on pedestrian flow and network design. Yamori (1998) examined macroscopic behavioral patterns in pedestrian crowds. Observing a large crosswalk, the focus was in determining how individual and collective behavior can be linked (i.e., how to translate from a macro to micro perspective). Mitchell and Smith (2001) use queuing theory to develop models for pedestrian network design. Weng et al. (2006) and Szemes et al. (2006) conduct simulation methods to model pedestrian movement and human walking behavior.

Automated pedestrian movement systems in airports are known to reduce passengers’ walking distance, but little is known about their effects on airport pedestrian flows. The effect of moving walks on pedestrian walking speeds is examined by Young (1999). Through survey data, Young found that there is no significant difference in the mean free-flow walking speeds with observed pedestrians’ characteristics within airport terminals. These characteristics include the pedestrian’s apparent age, the presence of baggage, the direction of travel, and party size. It also revealed that average free-flow walking speed is 264 ft (80.5 meters) per minute, approximately normally distributed with a standard deviation of 52 ft (15.9 meters). This result is very similar to Fruin’s study of 265 ft (80.8 meters) per minute.  Srinivasan (2009) discusses that people prefer to walk on moving walkways that move at low speeds, but find it advantageous to stand when walkways move at higher speeds. Also, he considers sensory conflicts, including visual flow information and leg/muscular information that the brain must resolve when using a moving walkway.

It is well known that passengers can often be distinguished by type or attribute, and knowing the relationship between passenger type and passenger conveyance use would be very useful. In fact on many attributes, Dresner (2006) notes that leisure and business passengers are very similar in terms of their choice of airport, their parking requirements, and the number of bags they check. In an unrelated study, Gates et al. (2006) examined how pedestrian walking speeds at intersections are influenced by signaling, traffic flow, and other pedestrians. Deegan and O’Mahony also examined how a new type of pedestrian waiting countdown timer influences pedestrian behavior at signalized crossings in Dublin. Iacono et al. (2008) proposed different types of destinations and accurately and robustly estimate distance decay models for auto and non-auto travel modes.

## Level of Service of Pedestrian Facilities

Airport terminal passenger mobility systems, such as moving walks, escalators, elevators, and APM systems provide more efficient ways to help air passengers reduce their walking distance and their walking time. However, we still need to consider the Level of Service (LOS) of pedestrian facilities.

The LOS concept was first developed in the field of traffic engineering in recognition of the fact that capacity design actually results in a certain level of planned congestion. Safety of pedestrian movement is necessary to be considered in all airports. Thus, the LOS concept should be used to assess the pedestrian’s efficiency in mobility and landside facilities (Omer and Khan 1988) in airports. Research work on pedestrian LOS design has its foundation in Fruin (1971), where a series of LOS design standards for walkways, stairways, and pedestrian queuing was developed. Fruin (1971) established measures of pedestrian effort and satisfaction based on the density of pedestrians in a corridor.

Walking speed, pedestrian spacing, and the probability of conflict in various traffic concentrations are the major factors that determined the breakpoints for the various service levels. Lee and Lam (2003) show LOS design standards for stairways in Hong Kong MTR stations, and they compare six LOS standards in Hong Kong stairways against LOS standards proposed by Fruin (1971).

Sarkar (1993) defined six service levels for pedestrians according to the quality of walkways in terms of safety, security, convenience and comfort, system continuity, coherence, and attractiveness. Similar to Sarkar (1993), Khisy (1994) found that these qualitative environment factors are just as important as the quantitative flow, speed, and density factor in planning and designing pedestrian facilities. In particular, both comfort and safety receive high importance in pedestrian decision making. Seneviratne and Morrall (1985) considered the perceptions of quality of service for the ranking and design of walkways. The findings of this article are based on the pedestrian studies conducted by Seneviratne (1985). Mori and Tsukaguchi (1987) conducted a study for evaluating the service levels of moving walks under different flow conditions in Osaka, Japan.

## Passenger Conveyance Systems

Moving walks, electric passenger carts, buses, and automated people movers are the most frequently used mobility technologies in airport terminal. An article by Leder (1991) presents comprehensive reviews of each of the above modes. Tough and O'Flaherty (1971) describe the operational details of the various types of moving walks. In addition, a comprehensive review of basic specifications of each installation is also included in this book. Kusumaningtyas and Lodewijks (2008) provide a literature review on accelerated moving walks (AMWs). In particular, they compare the characteristics of AMWs with other public transport systems--namely buses, light rail, Automated People Movers (APMs), and Personal Rapid Transits (PRTs). They conclude that accelerating moving walks can be competitive to the other short-distance transport modes. In addition, Al-Sharif (1996) and Smith (2005) have developed a great deal of information and comprehensive reviews of the escalators in actual operations.

However, we will focus on moving walks in this section. The conventional moving walk is a pedestrian-carrying device where passengers may stand or walk. Moving walk user safety aspects are discussed by Horonjeff and Hoch (1975). It is noted that traditional horizontal moving walks are restricted to a maximum speed of 180 ft. per minute and it would be more desirable to define capacity as the rate at which users can enter the moving walk and not the rate at which they exit. Young (1995) compares the moving walk with other primary modes of airport terminal passenger transportation.

The result shows that the average travel speed for passengers using the moving walk was only marginally higher than for those who chose to bypass the moving walk. Young develops a regression model to predict the travel speed and travel time of the passengers who have chosen to walk based on an empirical study of moving walks at San Francisco International Airport. He considered many passenger characteristics, including gender, luggage, normal walking speed, group size, etc. In addition, discrete choice models were developed to predict the probability with which passengers will choice to use moving walks (including the decision to walk or stand) or simply walk without assistance. In their study, the vast majority of passengers who choose the moving walks tended to walk instead of stand on the belt.

Joy (2001) presents a historic synopsis of secure vs. non-secure travel path issues at George Bush Intercontinental Airport/Houston, then examines non-secure inter-terminal passenger conveyance alternatives for the airport as a case study. Kyle (1998) conducts a study and presents a discrete-event simulation model to examine how existing and future operations would impact the mobile lounge fleets.

## Capacity of Conveyance Systems

The airport development reference manual (2004) indicates that the problem of traffic peaking at airports has been the subject of increasing concern by airline and airport operators around the world. And it recommends using schedule coordination to manage capacity demand. This manual gives comprehensive definitions of capacity in airports but not specific capacity numbers or estimates for conveyance systems in particular. Researchers have attempted to gauge the practical capacity of conveyance systems, with differing results across the studies. One clear theme does emerge, and that is manufacturer theoretical capacities can rarely be achieved in practice.

Pushkarev and Zupan (1975) state that human factors play a large role in defining the maximum capacity of an escalator.  They claim that a manufacturer rating of 50 persons per minute per foot of tread width (167 persons per minute per meter) cannot be achieved in practice. In this book, it suggests a maximum flow on a wide escalator (with steps designed for two people) to be about 18 persons per minute per foot (or 60 per minute per meter) with free arrivals and 27 persons per minute per foot (90 per minute per meter) under pressure from a waiting queue. Part of their findings was based on O’Neil (1974). In his study, he found that the maximum observed flow under crush conditions in subway stations was 103 pedestrians per minute on a wide escalator.  For design purposes, O’Neil (1974) recommends 90 persons per minute as the maximum value. O’Neil further emphasizes that the flow rate in the short-term is more realistic than any hourly extrapolation and should apply well whenever the flow is fed from a waiting queue.

Based on measurements at the Port Authority Bus Terminal, Fruin (1971) found that 31 persons per foot (103 per meter) of tread width per minute to be the maximum achievable capacity.  Also, Fruin (1971) calculated the maximum queue length at that rate of flow to be about 15 persons.

Barney (2003) conducted a comprehensive review of elevator and escalator capacity and flow. The author proposed a theoretical method of escalator capacity and found that an escalator with 39-inch (1m) nominal step width running at a rate speed of 1.64 ft (0.5 meters) per second has a theoretical handling capacity of 150 persons per minute. However, the author indicates that the practical handling capacity is about half of the theoretical (75 persons per minute) because the hesitations at boarding often result in an escalator not delivering its potential practical handling capacity.

Davis and Dutta (2002) estimated escalator capacity by using regression based on actual observations in the London Underground. They found that the capacity of an escalator at speed rate of 142 feet per minute, where passenger stood on both sides, would be approximately 108 persons per minute. The result is very similar to the findings in O’Neil (1974).

Pushkarev and Zupan (1975) and Davis and Dutta (2002) both state that the approach to escalator capacity and acceptable queue lengths are open issues. Based on the cited work, the maximum observed flow of an escalator is above 100 persons per minute. However, due to safety and LOS issues, a maximum flow on a wide escalator should likely be below 100 persons per minute.

## Automated People Mover Systems

In an airport environment, an APM is a mobility technology which transports passengers from terminal to terminal. It is an advanced transportation system in which automated cars operate on fixed guideways along an exclusive right-of-way. There are many research works which focus on how APMs contribute to each airport and its city (Austin 1993, Green 2001, Smith 1991, Willoughby, Winters 2006, Nicholas et al. 2001, Nicholas et al. 2001, Warren, Kunczynski 2000). Sproule (1985) and Sproule (1991) present comprehensive reviews of moving walks, buses, and APMs, and show a detailed description of the development of airports with people mover systems. These articles also describe the technology of each mode, performance measures, and inherent advantages and disadvantages.

ACRP 03-06 focused on improving and developing planning guidelines for automated people mover systems. This guidebook has been published as Report No. 37 – Guidebook for Planning and Implementing Automated People Mover Systems at Airports. As a result, we do not provide an in-depth discussion of the APM literature within ACRP 03-14.

# Catalog of passenger conveyance systems

## Planning Assumptions and Guidelines

Passenger conveyance systems in the airport environment, although not dissimilar to other facility uses, are unique in many ways.  This uniqueness comes in the way of understanding the implications of surge demand levels, passenger wayfinding while under stress, maneuvering multiple bags, interactions between floor plates of terminal facilities, the nuances of sterile and non-sterile environments, and the oftentimes 24-7 operational requirements placed on each of the components.  For these very reasons, it is critical to understand the baseline planning assumptions and established design guidelines for the application of these passenger conveyance systems.  These guidelines and assumptions range from understanding and applying varying capacities/levels-of-service, determining space requirements for the various components, and fully understanding the implications of life cycle costs with these systems.

    - Capacity/Level of Service

Capacity of a passenger conveyance system can typically be measured by the number of people served in a specified period of time.  An up escalator’s capacity has been measured in an airport environment to be about 60 people per minute (ppm) and a down escalator’s capacity has been observed in the same environment to have a capacity of 50 ppm.  Capacities of moving walk systems can be evaluated in a similar fashion as escalators.

Capacity of elevator systems can be measured in multiple ways, either in square-feet per person (sfpp) based on the elevator cab area and other space related criteria, or by the amount of waiting time a typical passenger experiences, which is a function of operational characteristics of the specific equipment.

Levels of Service (LOS) can provide an analytical basis for evaluating the sizing and locating of passenger conveyance systems.  However, there can be aspects of LOS that are more qualitative and subjective depending on engineering and planning judgment and on the local airport environment, since each airport owner can have different thresholds of acceptability in regard to service levels.  At present, there is no commonly accepted standard for either capacity of these conveyance systems or LOS criteria, although The Vertical Transportation Handbook suggests the elevator system provide average waiting times of less than 30 seconds in commercial buildings.  Simulation modeling can be useful in estimating system capacities and evaluating different LOS criteria.

    - Peak Flows and Time Periods

Capacity determinations involving periods of time (i.e., people per minute, passengers per hour) are typically evaluated in rounded time periods such as per second, per minute, per hour, or per day.  However, passenger conveyance systems in an airport environment experience different loading patterns than typical environments where these systems are in use.

For example, an escalator system in an airport terminal environment delivering passengers from their arriving flight to a baggage claim level below may handle a heavy loading period for a 5- to 15-minute time period.  In this instance, establishing a capacity level on a per-hour basis or longer is not relevant to the situation because the non-peak period for the remainder of the hour counteracts the peak passenger flow in determining capacity.  A per-minute capacity determination may not be appropriate either since the surged one minute flows over represent the passenger demands on the system.  As a result, a non-uniform time period for capacity determination (passengers per-peak 5 minutes, passengers per-peak 15 minutes, etc.) could be useful and more appropriate to the airport environment.

    - Passenger Space Requirements

Airport environments also introduce increased impact to space requirements for pedestrians due mainly to baggage and the area required for the attendant baggage.  A study at the Miami International Airport yielded the following results for pedestrian space.

Table 1. Passenger and Bag Space Requirement

* Excludes carry-on bag space requirement

The typical area required for pedestrians with various forms of baggage, excluding maneuvering space is illustrated below in Figure 1.  It should be noted that some studies also include additional interpersonal space to account for expected spacing between passengers.

## System Physical Characteristics

    - Elevators

Load capacities of the elevator systems range from 950 to 20,000 lbs.  Cab size and door size of the elevator systems can play a key role in determining system capacities and LOS for the chosen system but vary greatly across manufacturers.

The maximum speeds of the systems range from 75 to 1,600 feet per minute (fpm).  Systems on the lower end of this range are meant for buildings with a minimum number of stops or floors where speed is not of primary concern.  Systems on the higher end of this range are intended for use in high-rise buildings and are not anticipated to be needed in an airport terminal environment.  The maximum number of stops for an elevator system is closely tied to the maximum speed.  The more stops needed in a system, the faster the speed required in order to meet anticipated demand.

Figure 1. Typical Area Required for Standee

Maximum rise refers to the maximum height an elevator system can serve.  The maximum rises of systems shown above range from 12 feet to 1,600 feet.  The maximum rise is related to the propulsion or hauling system of the elevator.  Elevator hauling mechanisms can typically be divided into three categories:  traction, hydraulic, and climbing.  Each hauling mechanism provides different options regarding space needed for propulsion equipment, maintenance, maximum rise, and speed.

A specialized subset of elevator systems includes wheelchair lifts.  These systems are designed to move a wheelchair a shorter distance than typical elevators.  As shown in Table 2, the maximum rise of these systems ranges from 4 to 23 feet.  These systems typically accommodate only one person in a wheelchair at a time.  Maximum load capacities range from just under 500 lbs. to 1,000 lbs.

Table 2 shows system characteristics for elevators researched by manufacturer.

Table 2. Elevator Characteristics

| Manufacturer | Product/Model | Capacity (lbs) | Max Speed (fpm) | Max no. of stops | Max Rise (ft) |
| --- | --- | --- | --- | --- | --- |
| Mitsubishi | Diamond Trac | 2,000-3,500 | 200-350 | 10 | 98.5 |
| Abell Elevator International | Passenger Traction Simplex/Duplex | 2,000-4,000 | 100-350 | ** | ** |
|  | Holeless Hydraulic | 1,500-4,000 | ** | ** | ** |
|  | Passenger Oil Hydraulic Simplex | 1,500-4,000 | ** | ** | ** |
|  | Passenger Oil Hydraulic Duplex/Multiplex | 2,000-4,000 | ** | ** | 12 |
| Fujitec | Talon | 2,000-4,000 | 200-350 | 24 | 190 |
| Minnesota Elevator, Inc. | Geared Traction | 2,100-20,000 | 100-500 | ** | No Limit |
|  | Borehole | 2,100-20,000 | 200 | ** | 75 |
|  | Holeless (Twin Jack) | 2,100-20,000 | 200 | ** | 20-40 |
|  | Roped Hydro Twin Jack | 2,100-20,000 | 175-200 | ** | 100 |
| Schumacher | Hydraulic Passenger | 2,000-3,500 | 75-200 | ** | ** |
|  | Traction Passenger – Gearless | 2,000-3,500 | 500-1,200 | ** | ** |
| Stannah | Piccolo | 950-1,350 |  | ** | ** |
|  | Maxilift | 950-2,200 |  | ** | ** |
| ThyssenKrupp | SPF Traction | 2,100-4,000 | 200-500 | ** | ** |
|  | AC Gearless | 2,500-4,000 | 500-1,200 | ** | ** |
|  | AMEE C Series | 2,000-2,500 | 100-150 | ** | ** |
|  | AMEE Series | 2,100-3,500 | 80-150 | ** | ** |
|  | Conventional Series | 2,100-3,500 | 80-200 | ** | 79 |
| Schindler | 330A Hydraulic General Purpose | 2,100-4,000 | 100-150 | ** | 36-65 |
|  | 400A Traction General Purpose | 2,100-3,500 | 200-350 | 20 | 200 |
|  | 500A Traction General Purpose | 2,500-5,000 | 350-700 | ** | ** |
| Otis | Holeless Hydraulic | 2,000-3,000 | 100-125 | 3 | 20 |
|  | Telescopic Holeless Hydraulic | 2,000-3,500 | 100-125 | 5 | 44 |
|  | Roped Holeless Hydraulic | 2,000-3,500 | 100-150 | 7 | 60 |
|  | Holed Hydraulic | 2,000-3,000 | 100-150 | 7 | 60 |
|  | Gen2 Machine-Roomless | 2,100-4,000 | 200-400 | 30 | 196-300 |
| Kone | EcoSpace | 2,000-5,000 | 150 | 10 | 83 |
|  | MonoSpace | 2,000-4,500 | 200-500 | 27 | 230 |
|  | EcoSystem MR | 2,000-5,000 | 200-700 | 63 | 590 |
|  | Alta | 2,000-8,000 | 700-1,600 | 126 | 1,600 |

** Information not available

NOTE:  This information represents a sampling of product data from Manufacturer’s websites.  Additional information is available directly from the equipment manufacturers.

      - Cab Size

Cab size in an airport environment should be guided by several factors including the required system capacity measured in passengers per peak 15 minutes, specific use, area occupancies, passenger type (domestic/ international, business/leisure) and LOS to be achieved during the 15-minute peak period.  Specific uses include janitorial and supplies services, concessions goods, airport employee, airline personnel, public and travelling passenger conveyance.  With respect to passenger conveyance, cab size should be guided by the typical area required by standee with or without baggage, baggage carts, or wheelchairs.  Typical area occupancies may range between 1.5 sf/person for all female occupancy to 3.5 sf/person for an un-crowded elevator not including baggage.  With baggage, typical area occupancies may increase to 10 sf/person on average.  The cab size of the elevators researched above range between 40 and 55 square feet which results in an average of 4 to 5 persons with bags per cab.

      - Hauling System Performance

In the airport environment, LOS requirements should drive the required system performance of the elevators.  Hauling system performance directly translates into service times and thus LOS.  The elevators researched show a wide range of performance with maximum speeds from 75 fpm to 1,600 fpm, and maximum rise between 12 feet to 1,600 feet depending upon the system.

      - Door Size

Typical passenger elevator door sizes range between 3 and 5 feet.  Door size should be guided by required loading and unloading rates.  In the airport environment, the higher end of the range would be more suitable in order to provide for maneuverability for passengers with baggage and wheelchairs.  In a study for Ronald Reagan Washington National Airport, the elevator unloading rates for elevators with 5-foot doors for people without bags were 1.7 pps, and 1.4 pps for persons with bags.

      - Dwell Requirements

The quality of elevator service is normally based on waiting time.  A service time of 30 seconds and an average wait time of one (1) minute are typical in an airport environment. The service time corresponds to the time it takes to board the elevator and includes door open time, boarding/deboarding time, time to push the elevator button, and door close time.  Average wait time is the time a passenger waits for the elevator to arrive, prior to the start of the elevator service time.

      - Number of Floors Served

The number of floors served directly impacts the elevator round-trip time which is the time taken to service all floors.  In an airport terminal environment, the number of floors served will be limited by building height restrictions.  Hauling performance should be provided so as to meet required services times.

      - Boarding Areas

Boarding areas should provide adequate space for passenger queuing without extending into the circulation areas of the corridor.  Passenger queuing area occupancy averages 20 sfpp around elevator boarding areas in an airport environment. Furthermore, when baggage carts are to be accommodated, sufficient space for cart maneuverability must be also provided.

      - Cost Considerations

There are a number of cost considerations related to elevators.  Costs are directionally proportional to the size of the passenger cab, the number of floors served, the total lift, the speed of travel, the number of doors in the cab and the number of door panels in each door (bi-parting or cascading), the complexity of the controls and the type of lift system.

For example, hydraulic drives are less expensive while traction/cable hoist drives are more expensive. Costs related to installation within the building to accommodate hydraulic drive systems may increase depending on the circumstances surrounding the installation.  Piston-type hydraulic units are generally less expensive, but the accessibility to the site to drill the hole for the piston directly below the elevator shaft determines the cost effectiveness compared to the generally more expensive hole-less hydraulic drive system.  The cost of traction/cable hoist drive is less if the machine room can be located directly above the elevator shaft in the “penthouse” and more expensive with a configuration where the machine room below (i.e., machine room adjacent to shaft at or below the first floor served).

    - Escalators/Moving Walkways

In general, the capacity of continuous moving systems, escalators and moving walks is not a function of the number of steps/pallets or total standing area provided (i.e., manufacturer’s capacity).  Arrival characteristics and human capabilities will determine the practical or working capacity of these systems.  User boarding characteristics, combined with the expected traffic patterns and demand characteristics of the particular application, are the only true determinants of capacity and actual use.

There are however, a number of fundamental physical characteristics which should be understood and planned for by the designer.  These include speed performance, angle/maximum rise/maximum length, step width, and provided queue areas.  Each of these affects system performance and the ability to satisfy peak demands at airports.

      - Escalator Characteristics and Performance

Table 3 shows system characteristics for escalators researched by manufacturer.

Table 3. Escalator Characteristics

| Manufacturer | Product/Model | Max Speed (fpm) | Step Width (in) | Maximum rise (ft) |
| --- | --- | --- | --- | --- |
| Kone | ECO3000 | 100 | 24-40 | 49 |
| Otis | NCE Models | ** | 24-40 | 21.3 |
| Schindler | 9300 Advanced Edition | 100 | 24-40 | 24.6-55.75 |
| ThyssenKrupp | Velino | ** | 24-40 | 33 |
|  | Tugela | ** | 24-40 | 65.6 |
|  | Victoria | ** | 24-40 | 164 |
| Fujitec | GS8000 | 100 | 24-40 | 31 |
| Stannah | A2C/S/T | ** | 24-40 | 10-29.5 |
| Mitsubishi | J-type | 100 | 32-48 | 21.3 |
|  | Spiral 1200* | 80 | 40 | 11.5-21.7 |

* Rotation angle ranges between 103° and 170° (this angle measures the difference between the direction the passenger faces when entering this curved escalator and direction the passenger spaces when exiting the escalator).

** Information not available

Speed on virtually all escalators in an airport environment is consistent at approximately 100 fpm, whereas that rate can approach 130 fpm for moving walkways, due to safety element of the horizontal plane versus a rising plane.  Speed on escalators anywhere is largely regulated due to safety issues and hazards associated with each landing area.  For this reason, strollers, wheelchairs, carts, and small children are often directed to elevator conveyance devices.

      - Moving Walkway Characteristics and Performance

Table 4 shows system characteristics for moving walkways researched by manufacturer.

Table 4. Moving Walkway Characteristics

| Manufacturer | Product/Model | Max Speed (fpm) | Pallet Width (in) | Maximum Length (ft) | Max Angle of Inclination (if available) |
| --- | --- | --- | --- | --- | --- |
| ThyssenKrupp | Orinoco | 100-125 | 32-64 | 656 | 12° |
| Stannah | ST | ** | 32-55 | 440 | 10°-12° |
| Fujitec | GS8000 | 98 | 32-40 | 164 | 12° |
| Schindler | 9500 | ** | 32-40 | 328 | 12° |
| Otis | NCT | ** | 32-40 | 263 | ** |
|  | NPT | ** | 40-56 | 394 | ** |
| Kone | Autowalk Stanchion | 100-130 | 32-56 | ** | 3° |
|  | Autowalk Truss | 100-130 | 40-56 | ** | 3° |

** Information not available

Certain systems can be constructed with an incline.  Of the systems that included this information, the maximum angle of inclination ranged from 3° to 12°.Maximum speeds of the moving walkway systems researched range between 98 and 130 fpm.  Pallet widths for these systems ranged from 32 inches to 64 inches.  Maximum length varied from 164 feet to 656 feet.

A higher-speed moving walkway by ThyssenKrupp has been installed at Toronto Pearson International Airport. Over 900 ft long, the walkway operates at approximately 440 fpm for most of the length, with a speed closer to 110 fpm at the beginning and the end of the walkway. This walkway is a pallet-type in which the pallets "intermesh" with a comb and slot arrangement (see Figure 2 below from ThyssenKrupp). They expand out of each other when speeding up, and compress into each other when slowing down, with the handrails working in a similar manner.

Source:  Greater Toronto Airports Authority

Figure 2. Passenger Boarding High-Speed Moving Walkway

Source:  Greater Toronto Airports Authority

Figure 3. High-Speed Moving Walkway Installed Next to Conventional Moving Walkway
at Toronto Pearson International Airport

      - Angle / Max Rise / Max Length

The need to accommodate long escalators and moving walkway pathways is prevalent at airports.  This is attributed to the need for passengers to often transition between terminal and concourse floor plates to address the various service offerings, or to cover long walk distances.  For example, it is common practice among U.S. terminal designs to vertically separate the ticketing and baggage levels, thereby requiring a single floor change; however in instances where underground conveyances are utilized at larger facilities, these floor plate changes can be up to 5-6 floors in total.  This height further suggests a need for a maximum rise at airports of over 100 feet. Typical maximum rises seen among manufacturers varies from 10 feet to over 165 feet.  Rotational angle for escalators varies from between 103° and 170°.

      - Step Width

Escalator manufacturers rate theoretical capacity based on speed, assumed occupancy per step, and 100% utilization; however, many studies show that 100% utilization is never obtained.  Specifically, manufacturers use 5 people per 4 steps on a 32” wide escalator, resulting in an area occupancy of 2.7 sfpp, and 2 people per step on a 48” wide escalator, resulting in an area occupancy of 2.1 sfpp.  In contrast, observed capacity is generally 1 person per every other step on a 32” wide escalator and 1 person per step on a 48” wide escalator, resulting in an area occupancy of 4 sfpp.  Walking pedestrians do not change the capacity since the “boarding rate” is the capacity determinant, not the speed at which they move.  At airports which experience a higher percentage of pleasure flying or are of a tourist destination characteristic, a 48” wide surface is more accommodating of extra baggage.  Certainly in cases where a non-secure application is sought, and the average is more than 1.5 bags per person, the 48” wide escalator should be considered.

      - Queue Areas

The possibility that escalators and moving walkways can generate large queues, even at traffic levels below the capacity of the unit, is often overlooked by designers.  This is often generated by peak surges of passengers in an airport environment.  As a result, designers must often analyze the peak 10-15 minute period to account for pedestrian surges on conveyance devices.  Assessments of baggage quantities should be estimated as this will impact space requirements per person and per party.  Typical ranges of area queues ranges from 13 sfpp to as much as 17sfpp.  If programmed properly, this translates into standard queue platforms of between 15 and 30 feet in depth depending on the passenger surge, while the width of the queue mirrors the conveyance device width. The provision of a suitable queuing area at the entrance as well as an adequate safety zone at the exit end of the escalator must provide for sufficient space for the composite flows of all pedestrians moving through the area without hindering the movement to and from the escalator.

      - Cost Considerations

There are a number of cost considerations for escalators and moving walkways.  Cost are directly proportional to elevation of the unit, length of the unit, the location of the machine pit and potential structural considerations, the location of the unit itself and potential structural considerations, exposure to the elements, construction finishes of the unit.

Costs increase when the construction changes from anodized aluminum to stainless steel, and from metal panels to glass.  Also, escalators exposed to the elements have additional design and maintenance requirements, and may dictate an overhead cover of the unit.

    - Wheelchair Lifts

Wheelchair lifts are rarely seen in an airport environment, except in smaller terminal facilities or possibly at the apron boarding level.  Typically, wheelchair access to various floor plates in terminal facilities are satisfied by elevator conveyance devices.  This is principally due to the fact that wheelchair lifts are isolated devices oriented to one particular function and the resulting speed and capacities are significantly less than that of an elevator.  Table 5 below outlines the physical and performance characteristics of wheelchair lifts available in the market today.

Device capacity ranges from approximately 500 to 1,000 pounds.  The maximum speed varies from 10 to 30 fpm, while the maximum rise is less than 25 feet.

Cost considerations are similar to those of the elevator and escalators.

Table 5. Wheelchair Lift Characteristics

| Manufacturer | Product/Model | Capacity (lbs) | Platform Area (ft2) | Max Speed (fpm) | Max number of stops | Max Rise (ft) |
| --- | --- | --- | --- | --- | --- | --- |
| Savaria Concord | Profit SCL | 750-1,000 | ** | 30 | 5 | 23 |
| ThyssenKrupp Access | Porch-Lift Standard | 750 | 12-17.5 | 12-21 | 3 | 14.25 |
|  | Porch-Lift Toe Guard | 750 | 15 | 12-21 | 2 | 4.3 |
|  | Porch-Lift Enclosure | 750 | 15 | 12-21 | 3 | 14.25 |
|  | Porch-Lift Portable | 750 | 12 | 12 | 2 | 6.25 |
| Inclinator | SpectraLift | 750 | 12-15 | 20 | ** | 4.5 |
|  | Inclinator VL | 750 | 12-15 | 10 | ** | 12 |
| Garaventa Lift | Artira (Inclined) | 550 | 7-10.5 | 10-20 | 7 | ** |
|  | Xpress II (Inclined) | 495 | 7-10.5 | 13-16 | 1 | ** |
|  | Enclosure (Lift) | 750 | ** | 10-17 | 2-3 | ** |
|  | Shaftway (Lift) | 750 | ** | 10-17 | 2-3 | ** |
|  | OPAL (Lift) | 750 | ** | 10-17 | 2 | ** |
|  | STAAGE (Lift) | 750 | ** | 10 | 2 | ** |

** Information not available

# Data Collection

## Purpose

This project included data collection at five North American airports to obtain actual passenger usage and throughput of passenger conveyance systems.  A data collection plan was developed by the Team that identified the data to be obtained, airports proposed and visited for data collection, and the initial approach to be undertaken for obtaining passenger conveyance information at each airport. After the data collection plan was approved by the Panel, data collection dates were coordinated with each airport individually to ensure high passenger demand and to assist with necessary approvals for access in various areas of the airport terminals.

## Data Obtained and Data Collection Method

The data collection consisted of several types of data gathering to obtain the information on passenger conveyance usage, throughput rates, and other operational information:

- On-site time-and-motion studies:

  - Passenger choice of available conveyance options

  - Passenger walk vs. stand on escalators and moving walkways

  - Congestion on conveyance system (blocking ability to move freely)

  - Throughput, capacity or flow rate

  - Balking occurrences and waiting time delay and congestion when balking occurs

  - Number of bags/bag carts per passenger

- On-site observations:

  - Distance to alternative conveyance option

  - Visibility of and/or signage to alternative conveyance option

  - Speed of conveyance system

  - Measurement of various design characteristics (width of corridor, etc.)

  - Location of conveyance within corridor (walkway, passenger assist vehicle route)

  - Location and characteristics of conveyance options that support people mover systems

  - Number of elevators in bank

- Surveys/interviews with airport and staff and other information requests:

  - Airport CAD files (to measure available space adjacent to or between conveyance options)

  - Costs – capital (to include year of installation), maintenance, operating

Many items were collected with the use of personal digital assistants (PDAs) to record many data points.  The PDAs have on-screen displays to record data (such as number in traveling party, number of bags, airline, etc.) with a simple tap of the display screen (see Figure 4).  Once collected, the data was downloaded to another computer at the end of each data collection period to ensure no loss of data.

## Selection of the Five Airports for Data Collection

Figure 4. Sample of PDA Data Collection Screen

Various airport and passenger characteristics were considered in selecting the five airports for data collection on passenger usage/choice of conveyance systems, practical capacity, and airport/terminal design characteristics. The focus was on airports with enough traffic demand that passenger conveyance is a key issue.  However, some medium hub airports were considered due to the high usage of their conveyance systems.

Primarily, the airports were selected based on their passenger conveyance installations, but also with the intention of providing a reasonable representation of airport characteristics in the U.S. and Canada.  The airports represent a range of characteristics, including:

- Predominant passenger type – origin and destination (O&D) vs. connecting

  - Connecting passengers have limited time to make their departing flight, especially if their arrival was delayed for some reason.

- Predominant destination – international vs. domestic

  - Passengers on international arrivals will often have had longer flights (length of travel) and have higher demand for assistance on conveyance methods.

  - International passengers tend to have more bags per passenger than domestic furthering such a need.

  - International passengers exhibit different mode choices that vary with their terminal location and process. International passengers typically require more elevators than escalators due to the party size and number of bags, but initially upon deplaning, the most passengers prefer escalators as they only have carry-on baggage at this point. After clearing customs, these passengers will switch to needing more elevator than escalator capacity at the termination airport; however, if they are connecting after clearing customs, then they may desire either escalator (if re-check is on the same level as customs) or elevator (if re-check is on a separate level from customs) capacity.

- Purpose of travel – business vs. leisure

- Terminal configuration – centralized vs. decentralized

  - Passengers traveling through decentralized facilities may have less travel distance to their gates, but could have long transfer distances if the airport has significant amount of connecting passengers.

- Landside terminal – single vs. multiple

- Demographics – mixed vs. aging population vs. family destination

  - As the Baby Boomer population ages, the passenger conveyance requirements have likely increased and changed to more elevators; to collect data at airports that now have a high percentage of older adults will give a good indication to all airports as to what the conveyance requirements will be as the population ages.

- Seasonality of demand and predominance of excess equipment (e.g., large bags, skis, golf bags)

  - Some seasonal traffic to resort-type destinations has passengers with larger pieces of equipment:  baggage, skis, golf bags.

- Age of facility, age of conveyances

- Passenger conveyance usages – where are moving walks installed, operation of passenger assist vehicles, new emerging technologies, etc.

- New emerging technologies – airports that have installed express moving walks, sensor escalators, power ramps, etc.  The Team may decide to informally gather usage data from staff at airports and other facilities where implemented in order to supplement the collected data.

- Ongoing facility construction or redevelopment – the use of temporary facilities to work around construction projects during the anticipated data collection time-frame may impact passengers’ conveyance requirements or have closed certain conveyance facilities.  Depending on the construction phase and the number of facilities in operation, this characteristic can eliminate the facility from the data collection effort.

Collecting data across these characteristic airport types helped the Team determine if the different passenger types affect passenger conveyance needs or usage characteristics.  The resulting decision matrix for the data selection sites is shown in Figure 5.

Figure 5. Decision Matrix for Airport Data Collection Sites

The selected airports for passenger conveyance data collection are:

- Charlotte Douglas International Airport (CLT)

  - Vertical transitions between floors of terminal, mostly to bag claim.

  - Many moving walkways on secure side to facilitate connecting passengers.

  - Data collection split between secure and non-secure side.

- Dallas/Fort Worth International Airport (DFW)

  - Terminals are mostly on one level (Terminal D has domestic bag claim on lower level).

  - Vertical transitions used mostly to/from APM. Much passenger transfers between terminals.

  - Moving walkways connect terminals and also in Terminal D corridors.

  - Electric passenger carts used mostly for disabled passengers.

  - Most of data collection on secure side.

- Denver International Airport (DEN)

  - Multi-level terminal has vertical transitions between check-in, bag claim, etc.

  - Vertical transitions to/from APM.  Most connecting passengers transfer inside a concourse (not between concourses).  Most O&D passengers also use APM.

  - Moving walkways in Concourse B corridor; being added to Concourse C corridor.

  - Some data collection on non-secure side, but much on secure side.

  - Winter ski destination to consider effects of oversized luggage.

- Hartsfield-Jackson Atlanta International Airport (ATL)

  - Main terminal is mostly on one level.

  - Vertical transitions used mostly to/from APM.  Many passenger transfers between concourses.  Most O&D passengers also use APM.

  - Moving walkways in APM tunnel for those passengers who choose to walk between concourses.

  - Some conveyances in sterile area of Concourse E.

  - Most of data collection on secure side.

- Orlando International Airport (MCO)

  - Multi-level terminal has vertical transitions from garage to check-in, to bag claim, etc.

  - Moving walks connect to/from garage and also in concessions (to food court) area.

  - All of data collection on non-secure side.

  - Family/leisure destination with much luggage.

## Periods of Data Collection

The majority of North American airports experience their peak month of passenger traffic during the summer months.  However, this data collection was not dependent on having an absolute busiest time.  Busy hours can still occur at airports even though it might not be the peak month of passenger travel.  Enough of a sustained queue must be present so that the collectors could adequately measure throughput. Regardless of time of year, collecting data during busy hours provided enough data to indicate current conveyance usage at each airport.

Personnel at each airport were contacted to identify good data collection windows.  Local staff helped identify those weeks, days, and times of day that represent typical weekly or daily peak periods for the various facilities throughout the terminal complex.  If conveyances are typically used for connecting passengers, the days and times were different than for conveyances typically used by originating and terminating passengers.

The Team collected data for approximately four days at each airport.  This enabled the Team to observe demand at various flow points for each transition type and accumulate several hours of data.  For example, capturing passenger choice of vertical transitions in a parking garage required staging data collectors at the levels which have a direct link to the terminal to determine what percentage of garage users used each conveyance method offered.  Since more people are required to capture the total volume, we required several hours at each facility area to have sufficient data for a complete analysis.

## Airport Approval/Coordination

Discussions with each airport included what approvals or access were required for Team personnel to conduct the data collection.  Several areas identified for the data collection effort were located beyond the security screening checkpoint (circulation and gates).  Badging, temporary badging, and/or escort could be required for the Team to obtain access to the data collection areas, especially if multiple entrances/exits are required.  Where necessary, airport staff support helped coordinate with their airline tenants and government agencies such as the Transportation Security Administration (TSA) to ensure the data collection efforts went smoothly.  Each data collector carried a letter signed by Airport staff that authorized the data collection, in the event there were any questions from TSA, airport staff, or tenants regarding our work. Also, the Airports’ operations staff was asked to provide a considerable amount of information on costs and other items.

## Initial Data Collection Plan

Each airport had its own, specific data collection plan so that all of the special airport characteristic data is captured.  The following is a description of the initial plan for data collection:

  - Experimental Design.  Based on the data to be collected as discussed previously, identify the number of locations for data collection.  Consideration will be given to ideal times for peak- and off-peak data collection, as well as airport characteristic data that must be quantified.

  - Development of the Data Collection Survey.  For the quantitative data elements, develop standard wording and clearly defined start/end points for data to be observed.  For airport layout and dimensional data, identify the areas to be measured and the boundaries of the areas to be included.  For the quantitative data elements to be collected through observation, develop clearly defined categories of activities that must be observed and documented.

  - Program the PDAs and Access Database.  Develop and load the PDAs with the necessary survey instruments.  Develop the database that will become the repository for the individual data elements.

  - Schedule the Data Collection.  Identify the dates and times of day for data collection.  Confirm the schedule with the Airport and approvals/coordination.  Confirm the individual data collection team members and arrange their travel.

  - Initiate the Data Collection.  Meet with the data collection team on site at the airport.  Walk the facility with the Team to confirm the data collection locations and to resolve any inconsistencies in the data collection plan.

  - Collect Data.  The data collection team collects the necessary data.  At the end of each day, the data contained in the PDAs will be uploaded to one central computer where the data is reviewed to ensure there are no anomalies.  The data will be analyzed to determine if the estimated sample size is appropriate.  If more data is required, the data collection plan is developed/modified for the following day.

  - Data Collection Team Airport Report.  In addition to collecting data, the data collection team will prepare its airport specific notes.  These will include any special circumstances observed during data collection outside of the data collection plan.  These notes will be compiled and assembled with the results of the airport data analysis to form the airport report.

The factors influencing the numbers of data collectors at each airport site included the types of conveyances, the locations of the conveyances inside or outside the secure areas, the number of locations/areas to be covered, and especially the number of levels served by elevators. In addition, team size varied by individual day during the data collection.

## On-Site Observations at Each Airport

This section describes some of the specific observations during the on-site data collection. Note that the data analyses are detailed in the next chapter of this report.

    - HARTSFIELD-JACKSON ATLANTA INTERNATIONAL AIRPORT (atl)

The data collection occurred at ATL March 27 through March 30, 2009.  These dates were during the busy spring break travel season. Dates were also determined based on the availability of airport personnel to sponsor the visit since several members of the Airport staff escorted data collection teams. The Team had seven data collectors on-site collecting data both in secure and non-secure areas. Table 6 summarizes some of the relevant ATL characteristics, while Figure 6 displays the terminal layout.

Table 6. ATL Characteristics

| Terminal Type | De-centralized Single Terminal |
| --- | --- |
| Number of Concourses | 6 |
| Number of Gates | 199 (171 Domestic and 28 International) |
| Total Passengers | 88.0M (2009) |
| Originating Passengers | 40.72M (Domestic) / 4.39M (International) |
| Terminating Passengers | 38.34M (Domestic) / 4.44M (International) |
| APM | Plane Train – underground APM connects terminal building and concourses Sky Train – connects main terminal with Consolidated Rent-A-Car (CONRAC) facility |

Figure 6. ATL Terminal Layout

Approximately 90,000 data points were collected at ATL, with the following being the main areas of data collection:

- Escalator throughput down to the Airport People Mover (APM) and up from the APM stations.

- Choice of moving walkways or corridor in the tunnels for passengers not taking APM.

- Choice of escalator/stairs/elevator after security screening for departing passengers to APM.

- Choice of escalator or elevator to/from APM.

- Wheel chair storage.

- Some oversize (golf club) handling.

Escalators are the primary means of access to/from the APM in the main terminal and all concourses.  Figure 7 and Figure 8 show passenger density on the escalators.

| Figure 7. Escalators at APM Station | Figure 8. Three Escalators from APM at Main Terminal |
| --- | --- |

The Team observed several queues of passengers waiting to access elevators and/or escalators, as seen in Figure 9 through Figure 12. When passengers exit from a full APM train, there is a short walk distance from the train to the escalators in most of the concourses.  Due to the volume of passengers exiting the train, a queue develops at the escalator(s), which we observed to be 40 feet long in some instances.  When the next train arrived, the last passenger from the previous train was just boarding the escalator. (During peak periods of the year, we presume that there are times when passengers are still in queue for the escalator when the next train arrives at the station.) Due to this queue, the Team was able to obtain very good boarding rates for the escalators going up from the APM stations.

Figure 9. Passenger Queue for Escalator to APM

Figure 10. Passenger Queue for Escalator from APM Station

Figure 11. Passenger Queue for Escalator from APM

Figure 12. Passenger Queue for Elevator in Concourse

Wheelchair storage in the main terminal is shown in Figure 13 and Figure 14.  The newer wheelchairs in Figure 13 fit very close, requiring much less storage space than the more traditional wheelchairs depicted in Figure 14.

Figure 13. Wheelchair Storage Near Check-In (Delta)

Figure 14. Wheelchair Storage Near Check-In (Air Tran)

Figure 15. Storage for Traditional Wheelchairs

    - CHARLOTTE DOUGLAS INTERNATIONAL AIRPORT (CLT)

The data collection occurred at CLT July 22 through July 24, 2009.  These dates were during the busy summer travel season. The Airport provided visitor access each day (once a data collector exited the secure area, they could not re-enter). The Team had five data collectors on-site, collecting data both in the secure and non-secure areas of the terminal. Table 7 summarizes some of the relevant CLT characteristics, while Figure 16 displays the terminal layout.

Table 7. CLT Characteristics

| Terminal Type | Centralized Single Terminal |
| --- | --- |
| Number of Concourses | 5 |
| Number of Gates | 91 |
| Total Passengers | 34.53M(2009) |
| Originating Passengers | 16.16M (Domestic) / 1.09M (International) |
| Terminating Passengers | 16.20M (Domestic) / 1.09M (International) |
| APM | None |

Figure 16. CLT Terminal Layout

Approximately 33,000 data points were collected at CLT, primarily in the following areas:

- Moving walkways connecting the main terminal to the concourses and connecting between the concourses.

- Choice of moving walkways or corridor.

- Choice of escalator/stairs/elevator to bag claim.

Passengers using the many moving walkways in CLT are shown in Figure 17 through Figure 21.  A large portion of passengers choose to walk through the adjacent corridor rather than use the moving walkway, as shown in Figure 17. When the moving walkway is quite short, passengers may not perceive much benefit to using a walkway for a short distance.  Also, if there is congestion on the moving walkway, especially if passengers are standing with no space for others to walk past them, other passengers continue in the adjacent corridor as it is faster than standing.

Figure 17. Passenger Choice of Moving Walkway

Figure 18. Moving Walkways in the Middle of a Corridor

Figure 19. Moving Walkways on Side of a Corridor

Figure 20. Moving Walkways, Escalator, and Electric Passenger Cart

CLT has a corridor leading to Concourses D and E with one pair of moving walkways to/from Concourse E and a shorter pair of moving walkways to/from Concourse D. If a passenger going to Concourse E mistakenly boards the wrong walkway, they will miss their turn to Concourse E and have to continue on the walkway to D, then backtrack to their gate. Note the signage in Figure 21 showing the designation to Concourse E vs. Concourse D.  To the left of the moving walkways is a walking corridor. However, the columns and ceiling design make it difficult for a passenger to see the signs for the walking corridor when they first enter the corridor directly in front of the moving walkways. Given the layout, the moving walkways are most obvious to passengers entering this area, while walking corridor is an alternate mode; this is reasonable given the long distance to Concourses D and E. There are several concessions along the walking corridor; the Team observed several passengers that turn back to visit a concession after seeing it from the moving walkway.

Figure 21. Signage for Two Lengths of Moving Walkways

When wheelchairs are not being used (in between flights), they are often left in or near boarding lounges for easy access the next time one is needed.  Figure 22 shows several different types of wheelchairs used at CLT.

Figure 22. Wheelchair Storage Near Gate

    - DALLAS/FORT WORTH INTERNATIONAL AIRPORT (dfw)

The data collection occurred at DFW March 6 through March 9, 2009.  Two persons had DFW Airport security badges, and each badged person was able to escort five additional data collectors. The Team had six data collectors on-site, collecting data both in the secure and non-secure areas of the terminal. Table 8 summarizes some of the relevant DFW characteristics, while Figure 23 displays the terminal layout.

Table 8. DFW Characteristics

| Terminal Type | De-centralized with multiple terminals |
| --- | --- |
| Number of Terminals | 5 |
| Number of Gates | 152 |
| Total Passengers | 56.03M (2009) |
| Originating Passengers | 25.49M (Domestic) / 2.51M (International) |
| Terminating Passengers | 25.46M (Domestic) / 2.57M (International) |
| APM | Skylink (Secure Side APM connecting the terminals) |

Figure 23. DFW Terminal Layout

Over 14,000 data points were collected at DFW, with observations occurring mostly at the following areas:

- Escalators up to Skylink (APM) and down from Skylink.

- Moving walkways inside sterile corridors.

- Passenger choice of escalator, elevator, stairs immediately after deplaning from international flights.

- Electric passenger carts.

Moving walkways at DFW are depicted in Figure 24 through Figure 26. Several moving walkways are provided to reduce walking distance between terminals. However, the majority of passengers use the Skylink (APM) to connect between terminals. The few passengers that choose to walk between terminals often choose to maximize their walking, therefore choosing to walk the corridor rather than take the moving walkways. In fact, the majority of people on these walkways were employees. Also note in Figure 24 the number of electric passenger carts that park in this walking corridor to recharge. It is a convenience place for recharging since there is little passenger use in this corridor.

Figure 24. Moving Walkways between Terminals

Two different locations of moving walkways are shown below.  In the main corridor of international Terminal D, moving sidewalks are provided near the concession side of the corridor (as opposed to the boarding lounge side), as shown in Figure 25. Few people take the moving walkways. When asked, many passengers responded that they hadn’t even seen the moving walkways. Note that the corridor itself is very wide in this area. On the north side of this same terminal, a pair of moving walkways is provided along the side of the corridor, shown in Figure 26.  Again, very few passengers use the moving walkways. In both of these areas, the corridor width is very open and inviting such that walking is the primary mode while the moving walkways are provided as an alternative mode.

Figure 25. Moving Walkways in Center Section of Terminal D

Figure 26. Moving Walkways on Side of Corridor in Terminal D

The escalator and elevator access from a Skylink station is shown in Figure 27, with the elevator in the far distance (close-up of the same elevator shown in Figure 28). Escalators are the primary mode of vertical transition to/from the Skylink stations with elevators and stairs (in some locations) provided as alternative modes. Due to the distance to the elevators, most passengers do not perceive the elevators to be as convenient access. However, as seen at most airports observed, the only passengers taking elevators are those that require them due to mobility (wheelchair, stroller) or vertigo issues.

Figure 27. Escalators and Elevator in Skylink Station

Figure 28. Elevator at Far End of Skylink Station

In most Skylink stations, there are three escalators at each end of the station, with two in the “down” direction and one in the “up” direction. Only one is needed in the “up” direction since passengers arrive to the station at varying times. Two escalators are needed in the “down” direction to handle the surge of passengers all exiting a train at the same time. Having three escalators available also minimizes disruptions when an escalator is out of service for maintenance (Figure 29).

Figure 29. One of Three Escalators Out of Service

A variety of electric carts are used at DFW for transporting passengers and supplies throughout the airport terminals. Samples of the many types of electric carts are shown below in Figure 30.

Figure 30. Electric Carts at DFW

    - DENVER INTERNATIONAL AIRPORT (DEN)

The data collection occurred at DEN March 20 through March 23, 2009.  Data collection was conducted during the ski traffic season. The Airport worked through their security department to provide temporary badges each day to each data collector. The Team had six data collectors on-site, collecting data both in the secure and non-secure areas of the terminal. Table 9 summarizes some of the relevant DEN characteristics, while Figure 31 displays the terminal layout.

Table 9. DEN Characteristics

| Terminal Type | Centralized Single Terminal |
| --- | --- |
| Number of Concourses | 3 |
| Number of Gates | 93 |
| Total Passengers | 50.17M (2009) |
| Originating Passengers | 24.18M (Domestic) / 944,797 (International) |
| Terminating Passengers | 24.09M (Domestic) / 946,634 (International) |
| APM | The Train (Or Automated Guideway Transit System) |

Figure 31. DEN Terminal Layout

Approximately 60,000 data points were collected at DEN, with observations occurring mostly at the following areas:

- Escalator throughput down to APM and up from APM.

- Choice of moving walkways or corridor for connecting passengers (60” wide moving walkways).

- Choice of moving walkways or corridor on bridge to Concourse A.

- Choice of escalator/stairs/elevator after security screening for departing passengers to APM.

- Choice of escalator or elevator to/from APM.

- Some oversize (skis) handling.

- Electric cart storage.

In the main terminal, passengers pass through one of two passenger security screening checkpoints before proceeding down to board the APM to the concourses (Figure 32).  At each checkpoint exit, there are two escalators with stairs in between (Figure 33). This provided a good observation point to gather passenger choice of stairs, escalator, and walking on the escalator.

Figure 32. Collecting Data at the Security Screening Checkpoint Exit

Figure 33. Passengers on Escalators and Stairs to APM

The concourses at DEN have moving walkways in the middle of the corridor, as shown in Figure 34. The 60”-wide walkways are able to move a large number of people to/from the gates. Still many people choose to walk in the adjacent corridor rather than use the walkways.

Figure 34. Pair of Moving Walkways in DEN Concourse

Concourse B, serving United Airlines’ hub, has four 60” moving walkways with two in each direction (see Figure 35).

Figure 35. Concourse B with Two Moving Walkways in Each Direction

The 60”-wide moving walkways allow two or three people to board simultaneously and proceed down the walkway beside one another, as seen in Figure 36 and Figure 37.

Figure 36. Moving Walkways with Two People Abreast

Figure 37. Moving Walkway with Three People Abreast

DEN allowed the data collection team many opportunities to record passenger choice of walking in the corridor, walking on the moving walkway, or standing on the moving walkway (see Figure 38).

Figure 38. Passengers Choose Moving Walkways or Corridors

In the busy Concourse B, there are two escalators in each direction serving the APM station (see Figure 39). With the large number of passengers exiting the APM train, queues often form to board the escalator. When there is no queue and no one visible on an escalator, several people were observed to move to the right, expecting the escalator to be going in their direction of travel. But at some locations, the entry to the escalator is on the person’s left, and not their right side.

Figure 39. Two Escalators in Each Direction at APM Station

Figure 40. Queue Forms at APM Station Escalator

Electric passenger carts are stored and recharged in some hallways out of sight of passengers, as seen in Figure 41.  However, some carts are in unused boarding lounge areas, as shown in Figure 42.

Figure 41. Cart Storage and Recharging Area

Figure 42. Temporary Cart Storage in Boarding Lounge

    - ORLANDO INTERNATIONAL AIRPORT (mco)

The data collection occurred at MCO March 13 through March 16, 2009.  Data collection was conducted during the busy spring break season. All observations occurred on the non-secure area of the terminal. The Team had seven data collectors on-site. Table 10 summarizes some of the relevant MCO characteristics, while Figure 43 displays the terminal layout.

Table 10. MCO Characteristics

| Terminal Type | Centralized |
| --- | --- |
| Number of Terminals | 2 (4 concourses) |
| Number of Gates | 96 |
| Total Passengers | 33.69M (2009) |
| Originating Passengers | 15.33M (Domestic) / 1.48M (International) |
| Terminating Passengers | 15.38M (Domestic) / 1.49M (International) |
| APM | AGT (Secure Side APM connecting the concourses with main terminal) |

Approximately 40,000 data points were collected at MCO, with observations occurring mostly at the following areas:

- Escalator throughput down to baggage claim from APM.

- Choice of escalator/elevator for departing passengers when they first arrive to the airport.

- Choice of moving walkways or corridor from garage for departing passengers.

- Elevator boarding capacities.

- Some oversize (golf club) handling.

Figure 43. MCO Terminal Layout

Three large elevators are provided in the baggage claim area to transport passengers to ground transportation and parking.  As seen in Figure 44, queues often form at these elevators as groups of people depart baggage claim at similar times.

Figure 44. Queue at Elevators at Baggage Claim

Two escalators and stairs are available for passengers after they exit the APM to go to baggage claim.  With the surge of passengers exiting the APM, short queues may form at the escalators (see Figure 45), but quickly dissipate.

Figure 45. Queue Developing at Top of Escalators

With the large numbers of families visiting the theme parks in the Orlando area, the Team observed more situations at MCO with children having problems accessing the escalators. These often caused a few seconds delay for boarding at the top of the escalator. As seen in Figure 46, the picture on the left shows adults folding up the stroller while the child spends a few seconds getting comfortable before boarding the escalator. On the right, the child and adult first approached the escalator, but then balked to the stairs when the child did not want to board the escalator.

Figure 46. Children with Escalator Boarding Difficulty

With the surge of people approaching the escalators to baggage claim, many people would simply take the stairs rather than queue for the escalator. Figure 47 shows the bottom of the escalators and stairs where the passengers are exiting to baggage claim.

Figure 47. Escalator and Stairs to Baggage Claim

As passengers enter the lower level of the terminal, the signage directs them to one escalator for ticketing and hotel or another escalator for rental car return. To the left is signage for accessing the same terminal areas by elevator. Notice the last passenger in Figure 48 glancing at the sign before boarding the correct escalator.  The Team observed several parties where individuals would board the adjacent escalators, assuming they were going to the same destination. Then after a few seconds on the escalators, they would realize that one escalator is going to a higher level than the other elevator (see Figure 49), thus splitting the group.

Figure 48. Signage Directing Passengers to Specific Escalator

Figure 49. Escalators with Different Destinations

## Passenger Behavior

    - Passenger Difficulties with Escalators

At all five airports, the data collectors witnessed some passengers that had difficulty boarding or exiting an escalator. While this was not a regular occurrence (none of the events described below occurred more than five times), it is certainly worth noting here. Several issues were identified as causes for these difficulties:

- Crowding and congestion – When there were passengers having to pause and wait for other passengers to board the escalator, the waiting passengers could not properly prepare for the device. This was most often associated with a passenger boarding the device with baggage and not being able to see the device prior to boarding. Then there would often be added pressure for the passenger to move quickly as there were additional passengers behind him/her that were next in line.

- Families with small children – Children often have very limited experience with escalators, especially in an airport environment. Typically, the parent(s) would allow the children to go first or board simultaneously (which is the better approach from a safety point-of-view). If a child was apprehensive about the device, then this family would cause a momentary blockage at the device. On the other hand, if the parent(s) already boarded the escalator and the child refused to board, a more significant problem occurred. Parent(s) were now trying to go in the opposite direction of the device while the child stood at the entry to the device.

- Strollers – Strollers were regularly taken onto an escalator, and the impact was typically that they simply took up a bit more space. However, parent(s) were observed boarding the escalator with the child still in the stroller rather than looking for or taking an elevator. This is a major safety issue, especially in the downward direction. While some parent(s) successfully balanced the stroller (positioning themselves on the downward side of the stroller), we observed a few incidents where the child fell out of the stroller, and the resulting scene was chaotic. No injuries resulted from any of these incidents.

- Balance/fall - A passenger fell at the bottom of an escalator that caused the next few passengers to tumble over him before someone could get the escalator turned off.  For the average person, it is not obvious that turning off the escalator is a two-step process, thus even a few more seconds passed (and a few more passengers fell) before the escalator actually stopped.

- Signage - When escalators are provided in pairs, with both the up and down directions at the same platform, people have tendency to move to the right, expecting the escalator to be going in their direction of travel. But at some locations, the entry to the escalator is on the left side. The person may get to the first step of the escalator before realizing that it is not going in their direction of travel, then they must turn to proceed to the correct escalator. Note that this only happens to the first person walking toward the escalator, and then the others behind them easily recognize which escalator to board.

    - Passenger Use of Elevators

In most of the cases, elevators were used by those passengers requiring additional assistance, typically when the passenger was already being moved (horizontally) by a wheelchair. Many of the elevators were smaller and reserved for this use. In other locations (especially at MCO), larger elevators are strategically placed such that they are almost a primary mode of vertical transport. A significant number of MCO passengers use the elevators after they have collected their bags in bag claim.

Passengers in wheelchairs had assistance of another person, and there were no major issues in the boarding/deboarding process.  On occasion, a passenger would hold the door the open to allow another passenger to board, and thus increase the dwell time. Such incidents are infrequent and was not a major issue.

Many vertical transitions are now designed with the escalator as the primary mode. When passengers prefer an elevator, the line of sight and/or signage to the alternative conveyance should be obvious.  Some people will not take extra time to find the elevator option, as witnessed with parents taking a child in a stroller down an escalator rather than the elevator since it was some distance away and not visible from the escalator. In another instance, the signage for the elevator did not indicate the same destination as the escalator, causing confusion for a passenger.

    - Passenger Behavior During Off-Schedule Operations

During the 20 days of data collection, we did not observe a significant amount of off-schedule operations. However, during the ATL data collection effort, the train system was inoperative for a period of about two (2) hours on a Saturday between 11:00am and 1:00pm.  The impact was that all passengers were required to walk the tunnel in order to connect between concourses or reach the main terminal building. Fortunately, there are moving walkways positioned between each concourse in the tunnel and, in fact, this provided a significant increase in data collected around the moving walkways. This provided very good data on passenger choice regarding the moving walkways. While the train was out of service, we observed several passengers utilizing stray wheelchairs as baggage carts.

## Maintenance Costs

As part of the data collection effort, the Team also discussed maintenance costs of the various conveyance systems with airport staff.  Many airports have third-party providers maintain and support the systems.  These may be through contracts managed directly by the airport or by the tenant airline(s).  Some contract providers or airline consortiums were uncomfortable providing dollar amounts as they consider costs to be sensitive or proprietary information. Seattle-Tacoma International Airport (SEA) also provided maintenance costs as their staff had recently evaluated maintaining the older installed units compared to replacement costs. A summary of the maintenance cost information, per unit, obtained from the airports is displayed in Table 11. In addition, MCO provided an average annual maintenance cost of $399 per wheelchair lift. SEA evaluated maintenance costs of freight elevator separate from passenger elevators, with freight elevator maintenance ranging from $7,248 to $14,160.

Table 11. Passenger Conveyance Annual Maintenance Cost, Per Unit

|  | DFW | DEN | MCO | SEA |
| --- | --- | --- | --- | --- |
| Elevator | $3,319 | $14,235 | $12,194 | $3,564 - $15,108 |
| Escalator | $14,441 |  |  | $12,276 - $16,044 |
| Moving Walkway | ** | ** |  | $12,420 |

Due to the escalating corrective repair costs and unreliability of their aging equipment, SEA has started a replacement modernization capital program to update this infrastructure over the next four to five years at a cost of $80 million. This investment was based on the costs of the total airport corrective repairs, as seen in Figure 50.  The majority of this equipment is approximately 30 years old. It is interesting to note the rapid five-fold rise in maintenance costs in the last five years.

Source:  Seattle-Tacoma International Airport

Figure 50. SEA Escalator Annual Maintenance Costs

Information provided by DFW included a statement about Operations and Maintenance costs of operating an escalator being significantly higher than that of a two-stop hydraulic elevator (note that almost all DFW passenger elevators only carry people between two floors, thus have two stops). Since an escalator will consume electricity the entire time it is in service unlike a hydraulic elevator that uses nominal power unless running in the up direction. They estimate that even at maximum usage, this is only approximately 33% of the time. Escalators are also prone to more maintenance hours and more expensive damage repair, such as cuts in handrails and damaged steps. Operating escalators also carries a greater liability with significantly more recorded incidents than its elevator equivalent.

It should be noted that the elevator costs shown in Table 11 above cover a large range of elevator designs and drive systems.  The wide range of annual maintenance costs with the highest cost more than four times the lowest cost are indicative of the differences between hydraulic elevators at the lower end of the cost range and traction drive elevators at the higher end of the costs.  Also significantly impacting the annual maintenance costs is the number of levels served by the different elevators.

# Data Analysis

The data collected at each airport was input into a common database, then analyzed by type of conveyance and area of the airport.  This chapter summarizes this data analysis. All of the collected data is included in the database, described in the first chapter of this Report and the accompanying Guidebook, along with instructions to download the data.

## Escalators and Elevators

The data pertaining to escalators and elevators are analyzed to determine the average escalator board rates as a function of airport, direction of travel, escalator tread-width, and number of escalators; the average elevator board times as a function of airport and boarding type (boarding or de-boarding); elevator passenger characteristics; passenger vertical conveyance mode as a function of airport and direction of travel; and average number of roller bags as a function of airports and passenger vertical conveyance mode. Table 12 shows the average escalator board rates by airport, ranging from 31 to 39 persons per minute.

Table 12. Average Escalator Board Rates (in persons per minute) by Airport

| Airport | Board Rate |
| --- | --- |
| ATL | 31 |
| CLT | 34 |
| DEN | 39 |
| DFW | 32 |
| MCO | 33 |

Table 13 shows the average escalator board rates by airport and by up (U) and down (D) directions across all escalators at that airport. In the up direction, the boarding rates range from 33 to 43 persons per minute, while they range from 26 to 33 in the down direction. Escalator board rates for passengers going up are higher than the board rates when going down at all airports except MCO, where the up/down rates are identical. In general, it is believed that passengers may slow their board rate when going down as the entire device is not visible when boarding. At MCO, many of the down escalators are only transporting passengers one level, so the short distance is visible. Another reason the down escalators do not have as much capacity as an up escalator is the fact that passengers with wheel bags may find navigating the first tread a bit difficult as they negotiate the placement of the bag on the tread behind them whereas on an up escalator, the riser the passenger is standing on acts as wheel stop and does not require as much negotiation for the passenger.

Table 13. Average Escalator Board Rates (in persons per minute) by Airport and Direction

| Airport | Board Rate |  |
| --- | --- | --- |
|  | Up | Down |
| ATL | 34 | 28 |
| CLT | 35 | 32 |
| DEN | 43 | 28 |
| DFW | 36 | 26 |
| MCO | 33 | 33 |

Table 14 shows the average escalator board rates by airport and by tread-width. A higher board rate is observed for wider escalators in DEN, however the widest escalator at ATL does not deliver the highest boarding rate. In this case, the wide escalator may be located in an area where the passengers are moving at a slower pace entering the escalator.

Table 14. Average Escalator Board Rates (in persons per minute)
by Airport and Tread-Width (in inches)

| Airport | Board Rate (ppm) | Width (in) |
| --- | --- | --- |
| ATL | 25 | 36 |
| ATL | 43 | 40 |
| ATL | 30 | 48 |
| CLT | 34 | 40 |
| DEN | 35 | 40 |
| DEN | 45 | 48 |
| DFW | 32 | 48 |
| MCO | 33 | 38 |

Table 15 shows the average escalator board rates by airport and by number of escalators. The results clearly show an increase in board rate as the number of escalators is increased. However, this is not a linear increase, and it is dependent on the use and placement of the escalator bank.

Table 15. Average Escalator Board Rates (in persons per minute per escalator) 
by Airport and Number of Escalators

| Airport | Number of Escalators | Board Rate |
| --- | --- | --- |
| ATL | 1 | 39 |
| ATL | 2 | 28 |
| ATL | 3 | 28 |
| CLT | 1 | 34 |
| DEN | 1 | 55 |
| DEN | 2 | 34 |
| DFW | 1 | 36 |
| DFW | 2 | 26 |
| MCO | 1 | 38 |
| MCO | 2 | 32 |

As a final comprehensive view of escalator board rates, Table 16 shows the board rates by airport, number of escalators, direction of travel, and tread-width. It provides a sense of how variable the board rate can be for one, two or three escalators when considering all of the other factors related to this measurement.

Table 16. Average Escalator Board Rates (in persons per minute per escalator) by Number of Escalators,
Direction, Airport, and Width (in inches)

| Number of Escalators | Direction | Airport | Width | Board Rate |
| --- | --- | --- | --- | --- |
| 1 | Down | ATL | 36 | 35 |
|  |  | MCO | 38 | 38 |
|  |  | CLT | 40 | 32 |
|  |  | ATL | 48 | 36 |
|  |  | DEN | 48 | 49 |
|  | Up | MCO | 38 | 33 |
|  |  | ATL | 40 | 43 |
|  |  | CLT | 40 | 35 |
|  |  | DEN | 48 | 56 |
|  |  | DFW | 48 | 36 |
| 2 | Down | MCO | 38 | 32 |
|  |  | DEN | 40 | 18 |
|  |  | ATL | 48 | 25 |
|  |  | DEN | 48 | 28 |
|  |  | DFW | 48 | 26 |
|  | Up | DEN | 40 | 38 |
|  |  | ATL | 48 | 32 |
|  |  | DEN | 48 | 33 |
| 3 | Down | ATL | 36 | 19 |
|  |  | ATL | 48 | 19 |
|  | Up | ATL | 48 | 32 |

Table 17 shows the average elevator board times and passenger characteristics by airport and type of boarding (boarding or de-boarding).  Across all airport locations, the average time to board an elevator is always longer than the average time to deboard an elevator.

Table 17. Elevator Board Times (in seconds) and Passenger Characteristics by Airport

| Airport | Boarding or Deboarding | Avg of Pax | Avg Of Boarding Time Duration | Avg Boarding Time Per Person | Avg Of Large Bag | Avg Of Backpack | Avg Of Roller Bag | Avg Of Stroller | Avg Of Cart | Avg Of Golf Bag | Avg Of Wheel-chair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATL | Boarding | 2.44 | 10.04 | 4.12 | 0.05 | 0.44 | 0.62 | 0.30 | 0.08 | 0.00 | 0.50 |
| ATL | Deboarding | 2.19 | 7.10 | 3.24 | 0.03 | 0.26 | 0.57 | 0.22 | 0.09 | 0.00 | 0.44 |
| DEN | Boarding | 2.50 | 9.61 | 3.85 | 0.50 | 0.72 | 1.33 | 0.09 | 0.11 | 0.08 | 0.04 |
| DEN | Deboarding | 1.96 | 6.71 | 3.43 | 0.15 | 0.54 | 1.21 | 0.07 | 0.06 | 0.03 | 0.03 |
| DFW | Boarding | 2.63 | 8.58 | 3.27 | 0.04 | 0.08 | 0.21 | 0.13 | 0.17 | 0.00 | 0.00 |
| DFW | Deboarding | 1.06 | 3.94 | 3.71 | 0.00 | 0.00 | 0.25 | 0.19 | 0.13 | 0.13 | 0.00 |
| MCO | Boarding | 3.67 | 11.01 | 3.00 | 0.15 | 0.89 | 1.84 | 0.17 | 0.18 | 0.03 | 0.05 |
| MCO | Deboarding | 2.96 | 7.73 | 2.61 | 0.11 | 0.66 | 1.33 | 0.15 | 0.14 | 0.01 | 0.05 |

Table 18 shows the passenger vertical conveyance mode choice by airport. In all cases, the escalator is the preferred mode for a vertical transition, and those who use escalators predominantly stand on the device. Notice the much higher percentage of passengers using elevators at MCO. This is directly related to the placement and the available number of elevators. Passengers are clearly presented an elevator option in two key locations: (1) entrances to the terminal from rental car return and parking lots, and (2) in and around the baggage claim area.

Table 18. Passenger Vertical Conveyance Mode Choice (in percentage of persons) by Airport

| Airport | Percent Choose Elevator | Percent Choose Stairs | Percent Choose Escalator | Percent Walk on Escalator | Percent Stand on Escalator |
| --- | --- | --- | --- | --- | --- |
| ATL | 2.3% | 0.2% | 97.5% | 6.8% | 93.2% |
| CLT | 3.0% | 9.2% | 87.8% | 8.5% | 91.5% |
| DEN | 2.8% | 4.7% | 92.5% | 12.6% | 87.4% |
| DFW | 3.6% | 3.0% | 93.4% | 10.8% | 89.2% |
| MCO | 19.0% | 8.2% | 72.8% | 5.9% | 94.1% |

Table 19 shows the passenger vertical conveyance mode choice by airport and by direction of travel. This table provides a further breakdown of the results presented in Table 18.

Table 19. Passenger Vertical Conveyance Mode Choice (in percentage of persons) by Airport and Direction

| Airport | Direction | Pct Elevator | Pct Stair | Pct Escalator | Pct Walk on Esc | Pct Stand on Esc | Total Pax |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ATL | Up | 2.0% | 0.3% | 97.7% | 6.4% | 93.6% | 23,712 |
| CLT | Up | 1.3% | 11.8% | 86.9% | 10.5% | 89.5% | 2,280 |
| DEN | Up | 4.8% | --- | 95.2% | 12.3% | 87.7% | 5,284 |
| DFW | Up | 2.3% | 4.2% | 93.5% | 8.4% | 91.6% | 2,341 |
| MCO | Up | 30.7% | --- | 69.3% | 6.1% | 93.9% | 3,283 |
| ATL | Down | 2.7% | 0.1% | 97.3% | 7.2% | 92.8% | 20,698 |
| CLT | Down | 3.4% | 8.5% | 88.1% | 8.0% | 92.0% | 8,391 |
| DEN | Down | 1.6% | 7.5% | 90.9% | 12.9% | 87.1% | 8,808 |
| DFW | Down | 4.9% | 1.8% | 93.3% | 13.1% | 86.9% | 2,430 |
| MCO | Down | 16.9% | 9.7% | 73.4% | 5.9% | 94.1% | 17,830 |

While passengers regularly have a roller bag when using escalators for vertical transition, no more than 10% of these passengers would choose to walk (over simply standing) on the device to quicken their trip. This result was consistent across all airports. Table 20 shows the average number of roller bags by airport and by passenger vertical conveyance mode.

Table 20. Number of Roller Bags by Airport and by Passenger Vertical Conveyance Mode

| Mode Choice | Airport | Avg No of Roller Bags | Total Pax |
| --- | --- | --- | --- |
| Elevator | ATL | 0.26 | 1,032 |
|  | CLT | 0.36 | 317 |
|  | DEN | 0.46 | 397 |
|  | DFW | 0.40 | 172 |
|  | MCO | 0.69 | 4,017 |
| Stairs | ATL | 0.21 | 86 |
|  | CLT | 0.09 | 978 |
|  | DEN | 0.03 | 659 |
|  | DFW | 0.03 | 142 |
|  | MCO | 0.05 | 1,733 |
| Stand on Esc | ATL | 0.40 | 40,370 |
|  | CLT | 0.41 | 8,578 |
|  | DEN | 0.30 | 11,388 |
|  | DFW | 0.38 | 3,978 |
|  | MCO | 0.39 | 14,455 |
| Walk on Esc | ATL | 0.10 | 2,922 |
|  | CLT | 0.08 | 798 |
|  | DEN | 0.07 | 1,648 |
|  | DFW | 0.07 | 479 |
|  | MCO | 0.08 | 908 |

## Moving Walkways

The data pertaining to moving walkways are analyzed to determine the average board rates as a function of airport and moving walk tread-width, passenger horizontal conveyance mode as a function of airport, and average number of roller bags as a function of airports and passenger horizontal conveyance mode.

Table 21 shows the average moving walkway board rates by airport and tread-width. The 40”-width moving walkway was observed in three airports, ranging from 33 to 57 passengers per minute.

Table 21. Moving Walkway Board Rates (in persons per minute) by Airport and Width (in inches)

| Airport | Board Rate | Width |
| --- | --- | --- |
| ATL | 57 | 40 |
| CLT | 52 | 40 |
| DEN | 56 | 60 |
| DFW | 33 | 40 |
| MCO | 67 | 48 |

Table 22 shows the passenger horizontal conveyance mode choice by airport.

Table 22. Passenger Horizontal Conveyance Mode Choice (in percentage of persons) by Airport

| Airport | Pct Corridor | Pct MW | Pct Walk MW | Pct Stand MW | Total Pax |
| --- | --- | --- | --- | --- | --- |
| ATL | 35.6% | 64.4% | 91.0% | 9.0% | 6,632 |
| CLT | 47.3% | 52.7% | 85.3% | 14.7% | 11,841 |
| DEN | 30.4% | 69.6% | 85.8% | 14.2% | 19,004 |
| DFW | 45.2% | 54.8% | 91.3% | 8.7% | 2,923 |
| MCO | 28.7% | 71.3% | 70.7% | 29.3% | 4,886 |

It can be seen that more than half of all passengers will use moving walkways when given the choice. For those using the devices, a majority of the passengers choose to walk.

Table 23 shows the average number of roller bags by airport and by horizontal conveyance mode.

Table 23. Moving walk Mode Choice and Number of Rollers by Airport

| Mode Choice | Airport | Avg Of Roller Bags | Total Pax |
| --- | --- | --- | --- |
| Corridor | ATL | 0.33 | 2,362 |
|  | CLT | 0.39 | 5,597 |
|  | DEN | 0.32 | 5,770 |
|  | DFW | 0.26 | 1,322 |
|  | MCO | 0.52 | 1,403 |
| Stand on Moving Walkway | ATL | 0.28 | 383 |
|  | CLT | 0.25 | 915 |
|  | DEN | 0.26 | 1,882 |
|  | DFW | 0.29 | 139 |
|  | MCO | 0.58 | 1,019 |
| Walk on Moving Walkway | ATL | 0.29 | 3,887 |
|  | CLT | 0.38 | 5,329 |
|  | DEN | 0.29 | 11,352 |
|  | DFW | 0.31 | 1,462 |
|  | MCO | 0.49 | 2,464 |

## Summary of Other Database Components

The database allows users to view summary forms of vertical and horizontal conveyances at the study airports, as well as a planning tool for gauging transition equipment requirements when comparing a planned transition rate against observed transition rates and equipment performance at the five airports.  The reports are presented by conveyance types: elevator, escalator, and moving walkway, and available for each transition area and across all transition areas.

There are four options within the database: Planning Tool, View Data by Transition Area, View Data Across all Transition Areas, and View Equipment Detail in the Main Screen. While the Guidebook contains detailed information on the database and how to use it, this section provides a brief summary of each component.

    - Planning Tool

The planning tool is a decision-making framework to assist airport planners in deciding whether additional passenger conveyances are needed at the airport. Within the planning tool, it includes two main categories, vertical transition analysis and horizontal transition analysis. We use an estimated 5-minute peak flow, along with information provided by the user that defines the particular transition area of interest, to display an "anticipated passenger per minute requirement" compared to the "passenger per minute throughput achieved in our data collection at a similar transition point." Rather than suggest a specific transition equipment requirement, the database tool allows the user to quickly see the observed performance at the other airports where equipment was installed in a location similar to their proposed area.

    - View Data by Transition Area

This function allows the user to review the information of each subject by specific area within the airport. Users can choose several specific areas of interest, such as  parking and rental car, between parking/rental car and terminal, terminal to ticketing, ticketing to security, after security to concourse, after APM system, inside concourse, between concourses, inside terminal, terminal to baggage claim, after baggage claim.

Types of reports available at each transition area depend on the conveyance equipments used within the area, including the following statistics:

- Passenger mode choice and characteristics for vertical and horizontal conveyances

- Board rates for elevators, escalators, and moving walkways

    - View Data Across all Transition Areas

Users can review the data information across all transition areas by using this function. Users can choose among several specific measurements such as board rate, passenger characteristics, and mode choices. Reports are categorized by conveyance type – elevator, escalator, and moving walk.

- Five reports are available for elevators. All reports present elevator board rates and passenger characteristics. They are reported in five different views: by airport, by door open width, by airport and door open width, by elevator capacity, and by airport and elevator capacity.

- Seven reports are available for escalator board rate. The reports are presented in varied perspectives such as by airport, direction, width, number of escalators, and combinations of the mentioned factors. The escalator mode choice percentage reports provide percentage of passengers using stairs, elevators, escalators, escalator and walk, or escalator and stand. The reports are also offered by airport and travel direction.

- Three reports are available for moving walkways: 1) Moving walk board rate, 2) Moving walk mode choice, which provides the percentage of passengers using the corridor or moving walkway, as well as whether a passenger chooses to walk or stand once on the moving walkway, and 3) Average number of roller bags for each mode choice.

    - View Equipment Detail

In addition to information about the use of conveyance devices in airports, this database also provides the passenger conveyance details which include manufacturers of the conveyance systems and physical characteristics such as speed, length, and width. User can review each type of equipment within this section of the database.  Detailed information for elevators includes manufacturer, model, lower/upper capacity, lower/upper maximum speed, maximum number of stops, and lower/upper maximum rise.  Escalator information includes manufacturer, model, maximum speed, lower/upper step width, and lower/upper maximum rise.  Moving walkway information includes manufacturer, model, lower/upper maximum speed, lower/upper pallet width, maximum length, and lower/upper maximum angle.  Wheel chair lift information includes manufacturer, model, lower/upper capacity, lower/upper platform area, lower/upper maximum speed, lower/upper maximum stops, and maximum rise.

# Glossary of passenger conveyance system terms

| TERM | DEFINITION |
| --- | --- |
| Apron | A portion of an airport for parking, loading, unloading, refueling, servicing, or maintaining aircraft. |
| Automated People Mover (APM) | An automated horizontal transportation system that transports people in batches in an airport. |
| Baggage Cart | A small, wheeled vehicle that a passenger uses to transport baggage. |
| Baggage Claim Area | An area in a terminal where inbound checked baggage are delivered passengers. |
| Capacity | The number of people per unit time that a passenger conveyance system can transport. |
| Carry-on Baggage | Baggage that is taken by a passenger into the passenger compartment of an aircraft. |
| Checked Baggage | Baggage that is delivered by a passenger to an airline for transportation in the cargo compartment of an aircraft. |
| Computer-Aided Design | A design or drawing that is generated using a computer software. |
| Connecting Passengers | Passengers who arrive to an airport and depart from the same airport on a different aircraft with a different flight number. |
| Customs and Border Protection (CBP) | See U.S. Customs and Border Protection. |
| Deplanement | The disembarkation of a passenger or the unloading of a cargo item from an aircraft. |
| Design of Experiments | A efficient procedure for planning experiments in order to determine the effect(s) of one or more factors on a response variable. |
| Electric Passenger | An four-wheeled electrically-powered cart with a seating capacity of 3 to 10 passengers that is used to transport passengers requiring assistance. An electric passenger cart is also referred to as a golf carts, passenger assist vehicles or courtesy carts. |
| Elevator | A vertical transportation system that transports people or goods between floors in batches. |
| Enplanement | The boarding of a passenger or the loading of a cargo item onto an aircraft. |
| Escalator | An inclined conveyor transportation system that transports people between floors. |
| Golf Cart | See Electric Passenger Cart. |
| Level of Service (LOS) | A pre-defined standard with levels ranging from A through F that provides a quantitative specification for the quality of passenger service. |
| Maximum Rise | The maximum vertical distance that an elevator or a wheelchair lift may travel. |
| Moving Walk | A horizontal or inclined (up to 12 degrees) conveyor transportation system that transports people on the same floor or between floors. |
| Origin and Destination (O&D) Passengers | Passengers who begin or end their journey at a given airport. |
| Peak Hour | The hour that represents the peak demand for a given processing area. |
| Passenger Assist Vehicle | See Electric Passenger Cart. |
| Personal Rapid Transit (PRT) | An automated horizontal transportation system that transports small batches of people. Unlike an APM, a PRT does not have to stop at every station, reducing the travel time. |
| Queuing Theory | A mathematical theory that provides an analysis of a queue (or waiting line) and server system. Queuing theory includes derivations and formulae for the computation of the average time spent in a queue (waiting time) or in a system and the expected number of persons in a queue or in a system. |
| Secured Area | Portions of an airport, as defined in Airport Security Program, that provide passengers access to board an aircraft and to which access is generally controlled by the TSA, an aircraft operator, or a foreign air carrier. |
| Simulation | A technique for modeling the operation of a system over time. |
| Sterile Area | See Secured Area. |
| Terminal | A building in an airport that facilitates passenger enplaning or deplaning activities. |
| Transportation Security Administration (TSA) | An agency of the U.S. Department of Homeland Security that is responsible for securing U.S. transportation systems. TSA is the sole agency responsible for the screening of passengers and their checked and carry-on baggages. |
| U.S. Customs and Border Protection (CBP) | A federal law enforcement agency of the U.S. Department of Homeland Security that is responsible for securing and facilitating international trade and travel, and enforcing U.S. immigration and drug laws. |
| Wheelchair Lift | A vertical transportation system that transports a wheelchair and it's occupant between floors. The maximum rise is limited to 23 feet. |

# REFERENCES

Airport Cooperative Research Program 2010, Report 37 - Guidebook for Planning and Implementing Automated People Mover Systems at Airports, Transportation Research Board.

Al-Sharif, L. 1996, Escalator handling capacity: standards versus practice, London Underground Ltd.

Ashford, N., Stanton, H.P.M. and Moore, C.A. 1996, Airport Operations, 2nd edn, McGraw-Hill Professional.

Austin, T. 1993, "Traversing the terminals. (Cover story),” Civil Engineering (08857024), vol. 63, no. 9, pp. 40.

Bandara, S. and Wirasinghe, S.C. 1986, "Parameters of moving sidewalks for airport terminal pier fingers,” Transportation Planning and Technology, vol. 11, no. 2.

Bandara, J.M.S.J. 1989, Airport terminals-optimum configurations and gate position requirement, University of Calgary (Canada).

Barney, G.C. 2003, Elevator Traffic Handbook: Theory and Practice, Spon Press.

Davis, P. and Dutta, G. 2002, Estimation of Capacity of Escalators in London Underground.

Dempsey, P.S. 2000, Airport planning and development handbook: A global survey, McGraw-Hill.

de Neufville, R., Barros, d.A. and Belin, S. 2002, "Optimal Configuration of Airport Passenger Buildings for Travelers,” Journal of Transportation Engineering, vol. 128, no. 3, pp. 211-217.

Delve, D. 2004, "Up, Down - and Along Moving the Passenger,” Airports International Magazine, vol. 37, no, 1, pp. 26-27.

Dresner, M. 2006, "Leisure versus Business Passengers: Similarities, Differences, and Implications,” Journal of Air Transport Management, vol. 12, pp. 28-32.

Federal Aviation Administration 1994, Advisory Circular 150/5360-13 Planning and Design Guidelines for Airport Terminal Facilities, Federal Aviation Administration, Washington, D.C.

Fruin, J.J. 1971, Pedestrian Planning and Design, Elevator World, Inc.

Gates, T.J., Noyce, D.A. and Bill, A.R. 2006, "Are We Getting Slower? Updated Recommended Walking Speeds for Pedestrian Signal Timing,” Institute of Transportation Engineers, Milwaukee, WI, USA.

Green, R.E. 2001, "The Underground APM at Washington Dulles International Airport,” Proceedings of the Eighth International Conference on Automated People Movers, section 12, chapter 1, doi 10.1061/40582(2001)33.

Helbing, D. 1991, "A mathematical model for the behavior of pedestrians,” Behavioral science, vol. 36, no. 4; 4, pp. 298.

Hoogendoorn, S.P. and Daamen, W. 2005, "Pedestrian Behavior at Bottlenecks,” Transportation Science, vol. 39, no. 2, pp. 147-159.

Horonjeff, R. and Hoch, C.J. 1975, “Some Facts About Horizontal Moving Sidewalk at Airports,” Publication of: American Society of Civil Engineers, p.323-334.

Horonjeff, R.,McKelvey, F.X., Sproule, W. and Young, S. 2010, Planning and Design of Airports, 5th edn, McGraw Hill.

Hui, X., Jian, L., Xiaobei, J. and Zhenshan, L. 2007, "Pedestrian Walking Speed, Step Size, and Step Frequency from the Perspective of Gender and Age: Case Study in Beijing, China".

Iacono, M.J., Krizek, K.J. and El-Geneidy, A.M. 2008, Access to Destinations: How Close Is Close Enough? Estimating Accurate Distance Decay Functions for Multiple Modes and Different Purposes, University of Minnesota, Twin Cities ; Minnesota Department of Transportation.

International Air Transport Association 2004, Airport Development Reference Manual, 9th edn, Montreal-Geneva.

Jacobs, A. and Shiffrar, M. 2005, "Walking Perception by Walking Observers,” Journal of Experimental Psychology: Human Perception and Performance, vol. 31, no. 1, pp. 157-169.

Joy, J.E. 2001, "Non-Secure Inter-Terminal Passenger Conveyance Alteratives for George Bush Intercontinental Airport / Houston,” International conference on Automated People Movers.

Keegan, O. and O’Mahony, M. 2003, "Modifying pedestrian behaviour,” Transportation Research Part A: Policy and Practice, vol. 37, no. 10, pp. 889-901.

Khisty, C.J. 1994, "Evaluation of Pedestrian facilities: Beyond the level-of-service concept,” Transporation Research Record, vol. 1438, pp. 45-50.

Kusumaningtyas, I. and Lodewijks, G. 2008, "Accelerating Moving Walkways: A review of the characteristics and potential application,” Transportation Research Part A: Policy & Practice, vol. 42, no. 4, pp. 591-609.

Kyle, R. G. 1998, "Washington Dulles Internation Airport Passenger Conveyance Study,” pp. 1119.

Leder, W.H. 1991, "Review of Four Alternative Airport Terminal Passenger Modility Systems,” Transportation Research Record, vol. 1308, pp. 134-141.

Lee, J.Y.S. and Lam, W.H.K. 2003, "Levels of Service for Stairway in Hong Kong Underground Stations,” Journal of Transportation Engineering, vol. 129, no. 2, pp. 196-202.

Mitchell, D.H. and Smith, J.M. 2001, "Topological network design of pedestrian networks,” Transportation Research Part B, vol. 35, pp. 107-135.

Mori, M. and Tsukaguchi, H. 1987, "A new method for evaluation of level of service in pedestrian facilities,” Transportation Research, Part A, vol. 21, no. 3, pp. 223-234.

Nicholas, D.G., Solis, P.M. & Brown, D.K. 2001, "Airport on the Move,” Civil Engineering (08857024), vol. 71, no. 9, pp. 34.

Odoni, A.R. & de Neufville, R. 1992, "Passenger Terminal Design,” Transportation Research, Part A: General, vol. 26A, no. 1, pp. 27-35.

Omer, K.F. and Khan, A.M. 1988, "Airport Landside Level of Service Estimation: Utility Theoretical Approach,” Transportation Research Record, vol. 1199.

O'Neil, R.S. 1974, "Escalators in Rapid Transit Stations,” Journal of the Transportation Engineering Division, vol. 100, no. 1, pp. 1-12.

Pfurr, A. 2006, "Moving into the Future,” Airports International, vol. 39, no. 1, pp.36-37.

Pushkarev, B. and Zupan, J.M. 1975, Urban space for pedestrians, The MIT Press, Cambridge, Mass.

Russell, H. 2004, "Up in the air: Gatwick gets passenger moving,” Bridge Design and Engineering, no. 37, pp. 34-36.

Sarkar, S. 1993, "Determination of service levels for pedestrians, with european examples,” Transporation Research Record, vol. 1405, pp. 35-42.

Seneviratne, P.N. 1985, "Acceptable walking distances in central areas,” Journal of Transportation Engineering, vol. 111, no. 4, pp. 365-276.

Seneviratne, P.N. and Morrall, J.F. 1985, "Level of service on pedestrian facilities,” Transportation Quarterly, vol. 39, no. 1, pp. 109-123.

Seneviratne, P.N. and Wirasinghe, S.C. 1989, "On the optimal width of pedestrian corridors,” Transportation Planning and Technology, vol. 13, pp. 195-203.

Smith, L.L. 1991, "The Maturing Airport People Mover Field: Four Rounds of Eeperience at Tampa International,” Transportation Research Record, vol. 1308, pp. 130-133.

Smith, M.S. and Butcher, T.A. 2008, "How Far Should Parkers Have to Walk?" Parking, vol. 47, no. 4.

Smith, P.R. 2005, Elevators and escalators,Time-Saver Standards for Architectural Design, 8th edn, McGraw-Hill.

Sproule, W.J. 1991, "Airport Development with Automated People Mover Systems,” Transportation Research Record, vol. 1308, pp. 125-129.

Sproule, W.J. 1985, Planning of Intra-Airport Transportation Systems (People Movers, Moving Walkways, Bus Service), Michigan State University.

Srinivasan, M. 2009, "Optimal speeds for walking and running, and walking on a moving walkway," American Institute of Physics CHAOS, vol. 19.

Strakosch, G.R. and Caporale R.S. 2010. "The Vertical Transportation Handbook,” Fourth Edition,

Szemes, P.T., Korondi, P. and Hashimoto, H. 2006, "Human walking behavior model for intelligent space,” 2006 IEEE International Conference on MechatronicsIEEE, Piscataway, NJ, USA.

The Ralph M. Parsons Company 1975, "The Apron and Terminal Building Planning Manual."

Tough, J.M. and O'Flaherty, C.A. 1971, Passenger Conveyors: An Innovatory Form of Communal Transport, The Garden City Press Ltd., Letchworth, Hertfordshire.

Warren, R. and Kunczynski, Y. 2000, "Planning Criteria for Automated People Movers: Defining the Issues,” Journal of Urban Planning & Development, vol. 126, no. 4, pp. 166.

Wells, A.T. 2011, Airport Planning and Management, 6th edn, McGraw-Hill.

Weng, W.G., Chen, T., Yuan, H.Y. and Fan, W.C. 2006, "Cellular Automation Simulation of Pedestrian Counter Flow with Different Walk Velocities,” Physical Rewiew E, vol. 74, no. 036102.

Willoughby, W.D. and Winters, G.K. 2006, "Automated Access,” Civil Engineering (08857024), vol. 76, no. 10, pp. 74-81.

Wong, S.T. 1969, "A multivariate analysis of urban travel behavior in Chicago,” Transportation Research, vol. 3, no. 3, pp. 345-363.

Yamori, K. 1998, "Going with the flow: Micro-macro dynamics in the macrobehavioral patterns of pedestrian crowds,” Psychological review, vol. 105, no. 3, pp. 530-557.

Young, S.B. 1995, "Analysis of Moving Walkway Use in Airport Terminal Corridors,” Transportation Research Record, vol. 1506, pp. 44-53.

Young, S.B. 1999, "Evaluation of Pedestrian Walking Speed in Airport Terminals,” Transportation Research Record, vol. 1674, pp. 20-26.

Zacharias, J. 2001, "Pedestrian Behavior and Perception in Urban Walking Environments,” Journal of Planning Literature, vol. 16, no. 3.

Marlin Engineering.  January 3, 1999-January 8, 1999 Curbfront Survey, Technical Memorandum, January 1999.  Presented in Miami International Airport MIC/MIA Connector Baggage Transport Alternatives Study.  Lea + Elliott.  March 1999.

DCA New Terminal Project, Analysis of Baggage Cart and Vertical Circulation Requirements, JKH Mobility Services, February 12, 1992.